Explanation of this Uplink Dashboard view

The orange triangle with an exclamation mark indicates a runtime error returned by the SeqVal (sequence validation) service; left-clicking on it will pop-up a modal with details about the errors. In this particular case, the errors are about files in the /eng2 partition. Due to resource limitations, the /eng2 file system is not implemented in SSIM (surface simulation), thus any files that will be written there in flight will not be included in the ground simulation (flight software emulation technically) which validates (technically verifies) the integrated command load (meaning what already exists in flight and is/will be executing plus any additional new file and command uploads being sent during this tactical shift).