# Expected SSim WARNING EVRs and Known Behaviors

Mars 2020 Mission

Exported on 2026-05-03 20:44:15

Table of Contents

1	Disclaimer 1: This list is not comprehensive. It is being added to progressively, as test cases expose new WARNING EVRs.	3

2	Classifications:	4

3	Additional Warning Info:	15

4	Non-EVR But SSim-Related Idios	16

5	Expected Related Behaviors	17

# Disclaimer 1: This list is not comprehensive. It is being added to progressively, as test cases expose new WARNING EVRs.

2      Disclaimer 2: This is used for ground simulation only

# Classifications:

Idiosyncrasy: no negative impact expected to simulation

Unsupported functionality: function/behavior not implemented (or not fully implemented) in SSim

Bug: issue with function/behavior expected to simulate correctly

Non-SSim Idiosyncracy (NSI): Not specific to SSim, idiosyncratic behavior being correctly emulated

| Name/Message | Classification | Description | Reference (if applicable) |
| --- | --- | --- | --- |
| SEQ_EVR_DEACTIVATE_NOT_ACTIVE_NOT_SUSPENDED_ID (WARNING_LO) Attempt to deactivate Sequence File <seqid> failed because it is not active or in-stasis. | NSI | In master/submaster, expected when deactivating sequences that have already completed (e.g. submasters, cleanups, etc.) In OBP, expected for certain activities which still include 'nominal' cleanups and will run regardless of whether the activity runs long. |  |
| SEQ_EVR_CNTRLR_SEQ_OPEN_FAILED_WARNING (WARNING_LO) Sequence File <seqid> failed to activate because it could not be opened. | Usually NSI | Expected when activating sequences that don’t exist (e.g. out of the EKO) In restricted planning, the call to the next master will fail with this EVR. This is an idio and is not an issue as long as all masters are simulated successfully. |  |
| RCM_EVR_COL_CHECK_FAILURE (WARNING_LO) | NSI | This is expected when we point the RSM at the Cal-Target, which is affixed to the top of the rover body. This warning, therefore, is totally expected, and while it is real, it is acceptable when we are performing said Cal-Target activity. However, in the event that we are NOT performing a cal-target, this warning may be worth mentioning and will need to be dispositioned. |  |
| RCM_EVR_COL_CHECK_FAILURE (WARNING_LO) SCAM_EVR_LASER_COLLISION_CHECK_FAILED Laser collision check failed. Unable to fire the laser. (WARNING_LO) SCAM_EVR_SPECTRA_BEHAVIOR_ERRORS One or more errors occurred while executing the Spectra behavior. (WARNING_LO) | Ground Idio | This is NOT the same as the above actual NSI issue. This is ONLY okay in AEGIS sequences Because SSIM cannot know where AEGIS will be pointing, for the first target it will typically pick a default 0,0,0 coordinate that is benign, but subsequent find_target commanding will simulate as pointing to areas which may in simulation induce a rover collision model EVR warning. Until a workaround is implemented in SSIM, this is to be expected and will NOT happen ! We will not see these every time we use these sequences, but we can see them. | FTA-047 AEGIS Lite - see Activity Test Data Review, section 6. Same applies to FTA-048: AEGIS Heavy Future improvement of AEGIS modeling: MSTRIAGE-8724 - SSim: enhanced AEGIS support for target-finding |
| IMF_EVR_NOT_COMMANDABLE (WARNING_LO) and related instrument-level EVRS Including: SHERLOC_SPEC_EVR_NOT_COMMANDABLE PIXL_EVR_NOT_COMMANDABLE PIXL_EVR_MONITOR_NOT_COMMANDABLE SCAM_EVR_NOT_COMMANDABLE MOXI_EVR_NOT_COMMANDABLE SHERLOC_SPEC_EVR_NOT_COMMANDABLE Etc. | NSI (usually) | AS LONG AS these EVRs appear in cleanups, they are expected (cannot command instruments while they are already powered off). IF NOT IN A CLEANUP, these are a real issue, indicating some problem commanding the instrument. |  |
| SCAM_EVR_BU_THERMAL_OFF_FAILED SCAM_EVR_CHECKED_NOT_COMMANDABLE SCAM_EVR_MU_THERMAL_OFF_ERROR | NSI | Similar to above AS LONG AS these EVRs appear in cleanups, they are expected (cannot command instruments while they are already powered off). IF NOT IN A CLEANUP, these are a real issue, indicating some problem commanding the instrument. |  |
| CBM_EVR_REJECTING_MODE_CHANGE_IN_DORMANCY CBM_EVR_REQUEST_IGNORED SSIM Received failure during task deactivation (WARNING_HI) For CBM ONLY. | Bug | Needs investigation. When this shows up during shutdown: Known that this does not affect CBM operation; windows still execute. However, would still like to understand why we see these (and why it's only sometimes.) |  |
| MOT_EVR_NODE_OVERTEMP MOT_EVR_NODE_OVERTEMP_2 MOT_EVR_MOTOR_HAD_ERRORS | Unsupported Functionality | PRTs are not modeled in SSIM, so this combination of EVRs are expected. |  |
| NAV_EVR_HSM_REJECT_MSG NAV_EVR_ACTION_SKIP_IF_COMPLETE NAV_EVR_ACTION_COMPUTE_ODOMETER_USING_BACKUP NAVLIB_EVR_PLANNER_EVALUATE_ARC_SCALING NAV_EVR_DP_GET_DPH_ARGS_TOO_LARGE NAVLIB_EVR_PLANNER_EVALUATE_ARC_SCALING NAV_EVR_RELEASE_IMAGES_TOO_OFTEN | NSI, but check with RPs to make sure the particular instance is expected | This is a real FSW behavior associated with mobility. See the FR idio rule for details. | FR: MOBNAV-2019 |
| MOT_EVR_REQ_HAD_ERRORS MOT_EVR_EXER_HST_BAD_STATE | NSI | Behavior associated with SNC conops. | Per Julie Townsend: "Yes that is expected. And will be expected for any ACA activities that include station interactions for the forseeable future." |
| FILES_EVR_UNLINK_FAILED | Usually ground idio for mob | unlink failed for nav_world_map, errno 2 | Part of a nominal RP trav backbone setup strategy. If they are not intending to use the stored nav world map, they will reinforce the old one is deleted before driving/creating a new one. Because the nav world map is not a ground created product (similar to snapshot pcfg's), until we provide a predicted or downlinked copy of the world map to onboard list, this command will always fail whether or not one exists onboard. |
| FILES_EVR_UNLINK_FAILED | SSIM bug seen in constrained planning | unlink failed for path ________________ | The files on board predictions are supposed to be for the start of the state of the plan. However, they currently exclude the predictions from the seq_00100, which can cause an issue when in constrained planning. Benign if seen in constrained planning on sols prior to your current planning sol. |
| ACA_EVR_CHAMBER_RES_HALL_MISCOMPARE | NSI ...but check with ACA when this occurs to make sure the particular instance is expected. | Behavior associated with the SHA Bit Carousel. These are expected when cleaning up ACA campaign IDs. This is necessary after FTA-19 has been completed, to tell the ACA not to use the sealed witness tube as the current sample tube See SNC documentation in SNC Cacher for Sol 136 for related details. | Note that we see 30-50 of these EVRs when cleaning up ACA campaign IDs. SSim also produces 30-50 EVRs but only one is showing up in EVR Viewer due to MSTRIAGE-4768. |
| PIXL_EVR_POWER_TOO_EARLY or HELO_EVR_POWER_TOO_EARLY | NSI | This evr is benign, it was seen in SOX. Rover FSW will just wait a few extra seconds. It is benign as it and can be dispositioned on DL. If an instrument team or AL indicates they would like additional time, they can ask for the SIE to add a SEQ_WAIT_FOR 10 sec at the appropriate spot in the relevant submaster. |  |
| CBM_EVR_DELETE_FIND_FAIL | Usually ground idio | Integrated validation doesn't currently provide SSim with sequences based on the onboard list (instead we deliver all sequences used every Sol and those delivered sequences are provided to SSim). As a result of this behavior and the way our propagation toolchain works, in a multi-sol plan with a window delete in the first sol, cbm_00101 will try to run in the later sols. The window will have been deleted in the first sol, so it won't exist anymore and we'll get this EVR. Note this only affects VSE, as SIE procedure works around this. , we will run cbm_00101 on the first sol, it will be deleted from /eng/seq, and it will not be present on later sols. |  |
| NAV_EVR_USE_CACHED_DPH_NO_CACHED_DPH | Allowed ONLY during MOB_DUMP_KZ command | Allowable with this command, unknown if it's okay in other circumstances. Ref: Mobility Known Warnings & Dispositions |  |
| NAVLIB_EVR_TREES_RUN_ACE_ON_NODE_PATH_ROOT_INF_COST NAVLIB_EVR_TREES_FIND_BEST_PATH_ROOT_INF_COST NAVLIB_EVR_PLANNER_PROCESS_BEST_PATH_NO_RETREAT NAVLIB_EVR_PLANNER_FIND_BEST_ARC_NO_PATH NAVLIB_EVR_SELECT_ARC_NO_ARC_CMD | NSI | When using PREP_AVOID_ALL mode with MOB_GO_TO as the very first leg of the drive, warning lo's issued because it will not have a world map to start, the rover is expected it to stay in Directed driving for several steps at the start of the drive. See Mobility Known Warnings & Dispositions (search PREP_AVOID_ALL) for more info. As long as drive results in "woohoo" not an issue, else consult with RP. |  |
| RSM_EVR_REQ_PLAN_POINT_FM_FRAME2 RSM_EVR_REQ_PLAN_NULL SEQ_EVR_WAIT_CMD_COMPLETED_FAILURE Failed sequenced command opcode IMG_ACQUIRE ... | Ground idio | This is an idio when using AEGIS sequences. It is fallout from how FSW tracks the AEGIS target frame, and how SSim does not track that frame. This can happen if the last AEGIS run reflected in the NPM dump used to seed AEGIS resulted in failure to set one or more AEGIS frames on targets. In that case, the NPM dump will have the associated frames marked as invalid, resulting in these EVRs. |  |
| FM_EVR_INC_SITE_INDEX_UNUSED_SITE_FRAMES_FEW | NSI | FSW will autodeletes will occur as expected when needed. |  |
| SPAM_EVR_RUN_SEQ_FAIL_IGNORE | NSI | AS LONG AS these EVRs appear with this message: - "Warning: RUN_SEQ of SEQID054_has_not_been_set faileld." And SeqIDs: - 054, 055, 056, 057, & 058 It is expected as they are reserved for movie and mic hooks that are still in development. IF NOT THIS message or listed SEQIDs, it is a real issue, indicating some problem commanding. |  |
| SEQ_EVR_EXTRACT_CACHED_CHECKSUM_AND_SIZE_STATS_FAILED_ID | Usually ground idio | This occurs typically with SEQ_DELETE commands that fail due to SSIM not believing a given sequence is onboard the rover, resulting in that command's failure and the diagnostic EVR here. This is usually due to trying to delete a sequence that has not been used for more than a sol, coupled with the way we send sequences to SSIM, which does not use the onboard list for sequences, instead relying on a propagation rml which contains any onboard sequences. If you see this EVR and/or a SEQ_DELETE command failure outside of this context, then further investigation will be needed to confirm that the sequence is actually expected to be onboard . |  |
| SEQ_EVR_UNKNOWN_CMD_REPLY_ID | Can be NSI | This can but does not always happen when a sequence gets cut off. It is only "okay" if the sequence is expected to be cut off - otherwise it is an indication that something got cut off that shouldn't have been! | See SNC Known Warnings and Idiosyncrasies |
| evr's for SAPP propagation and TOD limit MOM_EVR_TWD_DRIVE_TIMER_EXPIRED MOM_EVR_FAULT DRIVE_EVR_STOP_MOM ACM_EVR_ERROR_ASSIGNED additional evr's for TOD limit ACM_EVR_ACTIVITY_FAILED_1 MOM_EVR_ACM_PRECHECK MOM_EVR_TWD_REJECT_NAV_DIRECTIVE_ARC_IMAGE | NSI | This can occur during Autonav driving that hits the SAPP Propagation limit "Mobility fault SAPP_MARGIN (first fault SAPP_MARGIN, drive fault NONE, nav fault NONE) in HSM state WAIT_TWD." or when the TOD limit is reached "Mobility fault CUTOFF_TIME (first fault CUTOFF_TIME, drive fault NONE, nav fault NONE) in HSM state WAIT_TWD." If you see these evr's outside of this context, it may indicate an unexpected problem, check with the RP's ARC_IMAGE evr can appear when attempting a drive step evaluation after cutoff time (won't happen as often) | See Mobility Known Warnings & Dispositions |
| MOM_EVR_TWD_MISSION_NODOM_INCONSISTENT | usually a ground idio | sometimes see that in SSIM next sol after a site frame increment | Idio: MOBNAV-2087 |
| PLANC_EVR_PLAN_RESTORATION_ERROR_WAKEUP_IN_FUTURE The current time is before the wakeup start time <time>. PLANC_EVR_MON_PLAN_ERROR_INVALID_ACTIVITIES Declaring plan error due to invalid configuration of activities during wakeup. | idio | Idio due to how we have to bootstrap propagation/"cut" the plan file in simple planner mode, to be specific. If they come any time after the very early part of the simulation, that's bad. |  |
| SSIM_EVR_FILES_FILESYSTEM_CHECK () not in rover filesystem sandbox | idio | SSIM idio, artifact of trying to handover from M/SM to OBP, since there is not way to specify runout plan. |  |
| FM_EVR_REJECTING_MSG FmHsm, Rejecting message with token TREE_GET_TRANSFORM. | idio | Currently identified as a benign SSIM idio | MSTRIAGE-9632 |
| PLANC_EVR_PLAN_RESTORATION_ERROR_INCONSISTENT_SHUTDOWN Inconsistent shutdown activity 1###### found when restoring plan. PLANC_EVR_MON_PLAN_ERROR_INVALID_ACTIVITIES Declaring plan error due to invalid configuration of activities during wakeup. | Idio | Similar to PLANC_EVR_PLAN_RESTORATION_ERROR_WAKEUP_IN_FUTURE: This is due to propagation of the inbound/ yestersol's plan simulating a shutdown in the future. This is harmless if it is appears before tosol's plan handover EVRs. |  |
| PLAN_EVR_FAILED_UPDATE_ACTIVITY_END_FIND Could not find an activity with id 5###### to update end (plan_global_data=-337100800). | Idio | Benign. Expected when tosol's plan is deactivated by the time PLANC tells PLAN to update end time of in-progress handover OBA. Due to a race condition between PLAN, PLANC, and the deactivation at the end of runout, but presents no actual concern. |  |
| NAV_EVR_ACTION_COMPUTE_UNCERTAINTY_DISTANCES_BAD_ODOM NAV_EVR_ACTION_FAULT_PREDECLARING_FAULT | Idio | Both benign when incrementing the site index. Due to S8.0.0.3 update: MOB_SET_ODOMETER command now calls NAV every time, which it didn't used to. Uncertainty is based on where the last Site Uncertainty breadcrumb was dropped, but here we just invalidated​ it (on purpose). Does not cause issues with SSim. |  |
| SSIM_EVR_ARM_TGV_CONSTRAINT_VIOLATED | ground idio | Per ActID-0136: High Tilt Sampling, these are expected given the higher sampling tilts and the macros are developed to consistently enforce rover tilt appropriate turret gravity vector constraints. It may or may not be produced, but if it is, this is an SSim-only EVR. May also be seen wrt to ActID-0144: Sample Dump for the similar reasons (tube vs gravity vector) Generally the RP's will fix this before an SIE would see it, unless there is a reason to allow it to pass through to integrated modeling, but it is not harmful. |  |
| RSM_EVR_REQ_CHECK_KIN_STATUS_FAILURE SGNC_EVR_RSM_KIN_INVERSE_DIR_NUM_SOL_INVALID RSM_EVR_REQ_PLAN_NULL MCAMZ_EVR_RSM_POINTING_FAILED MCAMZ_EVR_IM_FAIL_ACTION | NSI | For ZCAM Tau observations with the sun almost at zenith, there is a chance that the sun vector is collinear with the RSM mast (i.e. the sun is directly overhead the rover). This results in a failure to point the RSM due to kinematic singularity, and associated WARNING_HI/WARNING_LO EVRs. The activity should still successfully stow the RSM. Additionally, due to timing differences in SSIM the solar Tau activity may fail in SSIM but succeed on Mars. Therefore, we can safely ignore these EVRs in SSIM when associated with a ZCAM Solar Tau activity. | ZCAM Justification: ZCAM_Solar_Tau.txt |
| FILES_EVR_OPEN_INVALID PLAN_EVR_OPEN_FILE_FAILURE PLAN_EVR_LOAD_AND_VERIFY_ERROR Failed to load and verify plan file after reading 0 bytes (id=0, reason=OPEN). PLANC_EVR_ACTIVATED_RUNOUT | NSI | Expected during OBP handover in SSim because we do not have the next plan file onboard. Note that the warnings should show OBP failing to load and verify the Sol N+1 plan file after reading 0 bytes, and then successfully activating the corresponding runout plan file. These EVRs (along with failed plan setup sequence activations) should appear for each sol of runout included in the planning cycle. We do not expect to see these because the N+1 plan file should be onboard at handover!. Any plan file that fails to 'Load and Verify' due to any reason besides reading 0 bytes is not expected!. |  |
| PLANC_EVR_FAILED_ACTIVATING_PLAN PLAN_EVR_FAILED_UPDATE_ACTIVITY_END_FIND | NSI | Expected during OBP handover for the final sol of runout. Because neither the N+1 plan file or runout plan file is available in SSim on the final runout sol, OBP will fail to activate the new plan and the runout plan, then deactivate the active plan file. The final warning EVR in SSim should be the FAILED_UPDATE_ACTIVITY_END_FIND, which occurs when the Handover OBA is deactivated as part of plan deactivation. When PLANC reports the end status of the activity to PLAN, we see this nuisance warning as PLAN no longer has any memory of the deactivated plan or its activities. This should only happen if OBP drops into automode!. |  |
| ARM_EVR_DELETE_SAVED_NOT_FOUND | NSI | Expected during execution of ActID-0153. ARM_EVR_DELETE_SAVED_NOT_FOUND may be generated by the opposition-first sequences if the ActID's saved RSM pose arm target does not exist as we delete it unconditionally. This is expected and nominal behavior. |  |

# Additional Warning Info:

# Non-EVR But SSim-Related Idios

| Location | Issue Description | Disposition |
| --- | --- | --- |
| Seq Tree | When driving with RIMFAX, RFAX sequences show up as DEACTIVATED (RED) in the seq tree. | This is nominal for RIMFAX. Their sequence is supposed to run for the duration of a drive, but that can be variable. So it runs long, and the RPs deactivate it when they are done. The RPs say they expect to see this "whenever we drive". |
| Seq Report (Seq/Cmd Timing) | Failed sequence delete commands | SEQ_DELETE commands fail if the sequence being deleted is not "onboard” in the simulation run. In SSim, sequences are only onboard if they are a) delivered to the cycle or b) included in the modeling as pre-handover "propagation" sequences. This means that often, when deleting sequences used yestersol (through "Delete Next Plan" mechanism), the deletes succeed. However, when deleting sequences not used yestersol, the commands often fail in SSim. This failure has no negative impact on downstream modeling. |

# Other Expected Behaviors

| Command Name | Classification | Description | Example |
| --- | --- | --- | --- |
| SEQ_PAUSE_UNTIL_* fails | NSI | When a SEQ_PAUSE_UNTIL_* command is issued, but the PAUSE_UNTIL criterion is already true when the command is issued, the command will FAIL and the Diagnostic EVR "SEQ_EVR_PAUSE_EVAL_TRUE_COUNTDOWN_EXPIRED_ID" will be seen just before the modeled command fails. |  |