# Mars 2020 Acronym List and Glossary

Mars 2020 Mission

Exported on 2026-05-08 09:48:42

Table of Contents

1	General Mars 2020 Acronyms & Glossary	4

2	Simple Planner Acronyms & Glossary	58

This is a running list of 2020 acronyms and terms (aka jargon).  This has expanded to include definitions of non-acronym terms to have a one-stop shop for reference.

There is one table with general project terms, and a second table with Simple Planner-specific terms. The Simple Planner glossary is separated so that it can be more quickly and easily referenced during the roll out of the new functionality.

Feel free to add anything that's missing or edit anything that needs fixing.

# General Mars 2020 Acronyms & Glossary

| Acronym | Definition | Comments |
| --- | --- | --- |
| A/S | Aeroshell |  |
| ABA | Abrading Bit Assembly | Part of SCS. Bit assembly used for making surface abrasions with Drill. |
| ABLGA | Aft-pointing Backshell Low Gain Antenna | Al Chen: I don't believe this to be an accepted acronym. There are two X-Band LGAs on the entry vehicle: the Parachute LGA (PLGA) and the Tilted LGA (TLGA). |
| AC | Adventitious carbon | Adventitious carbon (AC) is a thin layer (~20-100ng/cm2) of organic hydrocarbons formed on clean metal surfaces. It is a sample contamination concern. |
| ACA | Adaptive Caching Assembly |  |
| ACM | Activity Constraint Monitor |  |
| ACI | Autofocus and Context Imager | For SHERLOC |
| ACE | Call sign for Flight Project Mission Controllers | Not actually an acronym, but a call sign for our illustrious team who uplinks commands, bundles of files, and sequences to our rover |
| ACS | Attitude Control System |  |
| Activity | Activity (used in the COCPIT Tool, also used by Simple Planner / On Board Planner) | - The planning unit that is used to model resources (power, data volume, duration, claims and parallelism) as well as generate sequences with auto-expansion. - Activities are defined in the Activity Dictionary (strategically defined and V&V’d as a part of the parcel) |
| Activity Group | Activity Group (used in the COCPIT Tool, also used by Simple Planner / On Board Planner) | - a container for a group of activities that occur sequentially and/or in parallel with each other - Developed strategically as a part of the parcel |
| AD | Activity Dictionary |  |
| AD | Arrival Date |  |
| ADAMS | Automatic Dynamic Analysis of Mechanical Systems | Multi-body simulation tool for mechanical dynamics |
| ADR | Adaptive Data Rate |  |
| AEDL | Approach, Entry, Descent, and Landing |  |
| AEGIS | Autonomous Exploration for Gathering Increased Science | Artificial Intelligence software to provide automated targeting for mast and eventually turret mounted instruments |
| AEGSE | Actuator Electrical Ground Support Equipment |  |
| AEPL | Atmospheric Entry/Powered Landing | JPL EDL simulation S/W Al Chen: This simulation was retired after use on MER. It was not used for MSL and will not be used for Mars 2020. |
| AFT | Allowable Flight Temperatures | This can be specified for operating or non-operating (survival) conditions |
| AG | Activity Group | APSS shorthand jargon for a planning unit that contains activities |
| AGL | Above Ground Level | Altitude measured from local ground level, not reference MOLA geoid |
| AHSE | Assembly, Handling and Shipping Equipment |  |
| AIDS | Assembly Instruction Data Sheets |  |
| AIS | Array Interface Slice |  |
| AITS | Action Items Tracking System |  |
| ALARA | As Low As Reasonably Achievable | Process for radiation protection |
| AMAN | Auto Maneuver |  |
| Ames | (not an acronym) | NASA Ames is named in honor of Joseph Sweetman Ames, a physicist and one of the founding members of NACA (NASA's predecessor) |
| AMMOS | Advanced Multi-mission Operations System |  |
| AMPCS | Advanced Multi-mission Operations System (AMMOS) Mission Data Processing and Control Subsystem |  |
| AO | Announcement of Opportunity |  |
| AOA | Angle Of Attack |  |
| AOS | Acquisition of Signal | Used in the DKF (DSN Keywords File) visualization of the DKF Report |
| AOTF | Acousto-Optical Tunable Filter | Optical device used in the SuperCam instrument |
| APAM | Activity Plan Approval Meeting | Resource report on power, data volume, etc. |
| APGEN | Activity Plan GENerator | Resource analysis tool |
| APID | Application Identifier | ID field in data products which says what type of activity spawned it |
| APL | Applied Physics Laboratory | Johns Hopkins University |
| APS | Active Pixel Sensor |  |
| APSS | Activity Planning Sequencing Subsystem |  |
| APXS | Alpha Particle X-ray Spectrometer | Instrument |
| AR | Activity Report |  |
| Arb | Arbiter | FSW system resource request handler (can grant/deny/rescind). Also the will of the Prophets. |
| ARC | Ames Research Center |  |
| ARC | Ambient Robotic Coring | Ambient testbed for coring using BBC-2, MSL brassboard arm |
| ART | Anomaly Response Team |  |
| AS | Aeroshell |  |
| ASC | Array Switch Card |  |
| ASIC | Application Specific Integrated Circuit | Now usually an FPGA |
| ASTTRO | Advanced Science Targeting Toolkit for Rover Operations |  |
| ATLO | Assembly, Test, and Launch Operations |  |
| ATN | Attenuator |  |
| ATP | Acceptance Test Procedure |  |
| ATS | Air Temperature Sensors | Part of MEDA - #1-3 mounted on RSM, #4-5 mounted on rover chassis |
| ATS | Autonomous Taget Selection | ats is a flight software component in flight software that supports AEGIS operation |
| AU | Astronomical Unit | Unit of distance equal to the mean earth-sun distance |
| Automode | Automode/Autonomous Mode | See Spacecraft Modes within this list |
| AVS | Avionics Subsystem |  |
| AWS | Amazon Web Services |  |
| AZ | azimuth | can refer to both a position in the sky or a motor |
|  | B |  |
| BAR | Bonus Application Rack |  |
| BAU | Business As Usual |  |
| BB5 | Bread Board 5 | Custom motor control board developed at JPL |
| BBB | Balance Bus Box |  |
| BC | Ballistic Coefficient |  |
| BC | Bit Carousel |  |
| BCB | Battery Control Board |  |
| BCE | Bench Checkout Equipment |  |
| BCH | Bose-Chaudhuri-Hocquenghem | Error control coding |
| BCR | Budget Change Request | Aka "Lien" |
| BCT | Boundary Conditions Testbed | Ambient testbed for coring using BBC-2, MSL brassboard arm |
| BCXS | Backshell Coaxial Transfer Switch |  |
| BDD | Baseline Description Document |  |
| BEG | Bridal Exit Guide |  |
| BIP | Backshell Interface Plate |  |
| BIST | Built-in Self Test |  |
| BLDT | Balloon Launched Drop Test | Parachute tests performed on Viking |
| BLGA | Backshell Low Gain Antenna |  |
| BMD | Balance Mass Device |  |
| BMJ-C | Balance Mass Jettison Device - Cruise |  |
| BMJ-E | Balance Mass Jettison Device - EDL |  |
| BoE | Basis of Estimate |  |
| BP | Backplane |  |
| BPM | Baseband Processor Module |  |
| BPP | Bogie Pivot & Pose | Mobility pivot on suspension, allows bogie motion & houses CFS for bogie pose during deploy. |
| BS | BackShell |  |
| BSC | Battery Switch Card |  |
| BST | Bore Sweep Tool | SCS tool to clean tube opening as needed to improve success of sealing. |
| BTE | Bench Test Equipment |  |
| BTP | Build to Print |  |
| BUD | Bridle, Umbilical, Descent rate limiter device |  |
| BUD-DI | BUD - Drop Item |  |
| BUHF | Backshell UHF antenna |  |
| BWTS | Backshell Waveguide Transfer Switch |  |
| BX | Bit Exchange | SCS Process/Activity. Process of exchanging bits between the Drill and the Bit Holders in the Bit Carousel. |
|  | C |  |
| C&DH | Command & Data Handling |  |
| C/S | Cruise Stage |  |
| CacheCam | Cache Camera | Camera assembly part of the Vision Assessment Station |
| CACHER | Current Activities, Concerns, Handovers and Events Reports | GDS Tool used for daily reporting similar to MSL Reports. |
| CAD | Computer Aided Design |  |
| CAD | Cruise Activity Description |  |
| CAM | Cost Account Manager Command Approval Meeting |  |
| CAMP | Campaign Analysis Mapping and Planning | Web-based geospatial tool for science operations. Based on the NASA AMMOS open source Multi-Mission Geographic Information System |
| CAMPCU | Camera and IMU Power Converter unit |  |
| CAP | Cost Account Plan |  |
| CAP | Characterization Activity Phase | MSL acronym. Renamed to Surface Operations Transition (SOX) on M2020 |
| CAP | Cruise Activity Plan |  |
| CAP | CIPA Auxiliary Pump |  |
| CAR | Catch All Rack |  |
| CAST | Control Analysis Simulation Testbed | JPL GN&C developed EDL simulation |
| CBA | Coring Bit Assembly | Part of SCS. Bit assembly used for drilling core samples. |
| CBE | Current Best Estimate |  |
| CBL | Cabling |  |
| CBLO | Core Break LockOut |  |
| CBM | Communications Behavior Manager |  |
| CC | Constant Current |  |
| CC | Contamination Control |  |
| CCB | Configuration Control Board |  |
| CCD | Charge Coupled Device |  |
| CCMD | Cache Component Mounting Deck |  |
| CCO | Command Constraint Override |  |
| CCSDS | Consultative Committee for Space Data Systems |  |
| CDH | Command and Data Handling |  |
| CDPR | ? Critical Design Peer Review ? |  |
| CDR | Clean Data Record | Supercam data record (clean calibrated data) |
| CDR | Critical Design Review |  |
| CDRL | Contract Data Requirements List |  |
| CDS | Circuit Data Sheets |  |
| CDP | Center Differential Pivot | Mobility pivot located on top deck. Relates the motion of port/starboard suspension halves. |
| CEB | Compute Element Backplane |  |
| CEDL | Cruise, Entry, Descent, and Landing |  |
| CEPCU | Compute Element Power Converter Unit |  |
| CFA | Cumulative Fractional Area | Defines how much of an area is covered by rocks and of what size |
| CFC | Chloro-Fluro Carbon |  |
| CFD | Computational Fluid Dynamics |  |
| CFDP | CCSDS File Delivery Protocol |  |
| CFS | Constant Force Spring | Component of Bogie Pivot & Pose used to rotate bogie wheels flat and damp oscillations shortly before touchdown |
| CFTP | CCSDS File Transfer Protocol |  |
| CFU | Colony Forming Unit |  |
| CG | Center of Gravity |  |
| CGID | Cruise GNC Interface Drivers |  |
| CHE | Camera Head Electronics | For SHERLOC |
| CHIMRA | Collection and Handling for In-Situ Martian Rock Analysis |  |
| CHRS | Cruise Heat Rejection System |  |
| CI | Campaign Implementation |  |
| CI | Continuous Integration |  |
| CICT | Computing, Information, and Communications Technologies (Program) |  |
| CIPA | Cruise Integrated Pump Assembly |  |
| CL | Component Library | APSS shorthand jargon for a top-level planning unit |
| CLF | Close Loop Flyaway |  |
| CLGA | Cruise Low Gain Antenna |  |
| CLR | Closed Loop Receiver |  |
| CLTU | Command Link Transmission Unit |  |
| CM | Configuration Management |  |
| CM | Center of Mass |  |
| CMC | Centralized Motor Controller (card) |  |
| CMD |  | Command |
| CMGA | Cruise Medium Gain Antenna |  |
| CMM | Coordinate Measuring Machine |  |
| CMM | Capability Maturity Model |  |
| CMM-I | Capability Maturity Model - Integrated (w/Sys Engr) |  |
| CMSA | Cruise Mission Support Area |  |
| CNF | Conjunctive Normal Form | Representation used to express OBA dependencies |
| COA | Council of Atmospheres |  |
| COCPIT | Component-based Campaign Planning, Implementation, and Tactical |  |
| COFR | Certificate of Flight Readiness |  |
| Cog-E | Cognizant Engineer |  |
| Co-I | Co-Investigator |  |
| Component |  | - A group of one or more Activity Groups meant to satisfy some intent. Defined and strategically managed/V&V’d in the Component Library. |
| Constraint |  | Added to OBAs in a plan to constrain timing or ordering (dependencies) - Timing constraints (earliest start time, latest start time, relative to geometric times like noon, sunrise, sunset) - Dependency constraints (Start X before Y Starts, Start X after Y Ends, Do X during Y) |
| Copilot |  | Capability within COCPIT that generates a schedule of activity groups (OBAs), pre-heats and maintenance heating, CPU activities that satisfy all constraints. The scheduling algorithm is based on the FSW Onboard Planner algorithm. - Copilot is used in both Master/Submaster planning mode (current paradigm) and in Simple Planner planning mode to help the planning team understand what may happen on the rover based on plan contents and constraints |
| COPS | Cruise Operations |  |
| COPV | Composite Overwrapped Pressure Vessel |  |
| COTS | Commercial Off The Shelf |  |
| CPAM | Cruise Power and Analog Module | Major Avionics Assembly |
| cPCI | Compact Peripheral Component Interconnect | Data Bus |
| CPR | Cybersecurity Patch Release |  |
| CPT | Comprehensive Performance Test |  |
| CQCM | Cryo Quartz Crystal Microbalance |  |
| CR | Concept Review |  |
| CR | Change Request |  |
| CRC | Cyclic Redundancy Check | Data error detection technique |
| CRC | Critical Relay Control |  |
| CRF | Command Reply Frame |  |
| CS3 | Common Software and Services Subsystem |  |
| CS | Cruise Stage |  |
| CS | Conducted Susceptibility |  |
| CSA | Cruise Solar Array |  |
| CSD | Cruise Shunt Driver |  |
| CSL | Campaign Science Lead | Operations role, pronounced "sizzle". |
| CSLA | Cruise Shunt Limiter Assembly |  |
| CSM | Cross String Communication Module |  |
| CSPL | Chemical Separation & Processing Laboratory |  |
| CSR | Cruise Shunt Radiator |  |
| CSS | Cruise Stage Separation |  |
| CSSO | Common Software and Services Subsystem Single Sign On |  |
| CSTAGE | Cruise Staging Venue |  |
| CSV | Comma-Separated Values |  |
| CTM | Contract Technical Manager |  |
| CTS | Coaxial Transfer Switch |  |
| CV | Constant Voltage |  |
| CVCM | Collected Volatile Condensed Mass |  |
| CWAWS | Contractor Work Authorization/Work Summary |  |
| CWL | Continuous-Wave Laser | For focusing, and a microphone. |
| CWS | Common Workflow Service |  |
| CWTS | Cruise Waveguide Transfer Switch |  |
| CXS | Coaxial Transfer Switch |  |
|  | D |  |
| D/L | Downlink |  |
| D/S | Descent Stage |  |
| DAL | Differential Adjustable Linkage | Adjustable linkage connecting Main Differential Pivots to Center Differential Pivot |
| DAN | Dynamic Albedo Neutrons | Instrument |
| DAT | Product DATa file | Data product from the rover |
| DARTS |  | Spacecraft dynamics simulation tool |
| DBA | Data Bus Adapter |  |
| DBA | Drill Bit Assembly | MSL hardware that regolith passes into when drilling |
| DBA | Drillable Blank Assembly | M2020 hardware; a can of material sealed with foil that can be drilled into, mounted near SHERLOC cal target |
| DBAT | Data Bus Adapter w/Termination |  |
| DBE | Double Bit Error |  |
| DC2 | Design Cycle 2 | May 2002, aka Minimum Mission Design Cycle |
| DCD | Data Capture Delivery | DSN Data Flow |
| DCE | Descent Compute Element |  |
| DCO | Data Cut Off | NAV terminology for the time when they receive the last data used for the orbit determination process |
| DCXS | Descent Stage Coaxial Transfer Switch |  |
| DD | Displacement Damage |  |
| DDC | Descent stage Downlook Camera |  |
| DDI | Defined Data Item | Used for conditional sequencing |
| DDOR | Delta Differential One-way Range | Radiometric navigation |
| DDP | Development and Deployment Plan |  |
| DDP | Defect Detection and Prevention (Tool) |  |
| DDR | Detailed Design Review |  |
| DDUT | Drop Dead Uplink Time |  |
| DE | Detector Electronics |  |
| DEA | Descent Engine Assembly |  |
| DEA | Digital Electronics Assembly |  |
| DEM | Digital Elevation Map |  |
| DET | DETailed | Abbreviation of the word "detailed," in context example, MCA DET Record |
| devTT | Development Threadlet |  |
| DFE | Direct From Earth | Telecom related |
| DGB | Disk Gap Band | Parachute configuration |
| DGID | Descent Stage GNC Interface Drivers |  |
| DHMR | Dry Heat Microbial Reduction |  |
| DIM | Discrete Interface Manager |  |
| DIMU | Descent Stage IMU |  |
| DISCO | Discrete Output |  |
| DITL | Day In The Life |  |
| DLA | Declination of Launch Asymptote |  |
| DLGA | Descent Low Gain Antenna |  |
| DM | Data Management |  |
| DMA | Direct Memory Access | Data transfer without CPU involvement |
| DMC | Distributed Motor Controller |  |
| DMCA | Descent Motor Control Assembly |  |
| DMD | Data Monitor Displays |  |
| DMX | Data Managment and Transmission |  |
| DN | Digital Number |  |
| DOE | Department of Energy |  |
| DOF | Degree of Freedom |  |
| DOM | Domain Object Management | The staging area where SCMFs are stored in prep for uplink to the flight system. |
| DOSA | Deactivate Other Sequences All |  |
| DOY | Date of Year |  |
| DP | Data Product |  |
| DPA | Descent Power Assembly | Major Avionics Assembly |
| DPAM | Descent Power and Analog Module |  |
| DPBNJ | Descent Stage Power Box and Junction |  |
| DPFA | Descent Pyro Fire Assembly |  |
| DPLR | Data Parallel Line Relaxation | 3-D structured, finite-volume Navier-Stokes solver |
| DPO | Data Product Object |  |
| DPRA | Descent Pyro Relay Assembly | Major Avionics Assembly |
| DPSR | Data Product Summary Report |  |
| DRAM | Dynamic RAM |  |
| DRD | Directors' Review and Discussion |  |
| DRL | Descent Rate Limiter |  |
| DRM | Design Reference Mission (Rover Expected Performance) |  |
| DRT | Dust Removal Tool |  |
| DS | Descent Stage |  |
| DSA | Digital Sun sensor Assembly |  |
| DSB | Descent Stage Battery |  |
| DSE | Digital Sun sensor Electronics |  |
| DSENDS | Dynamics Simulator for Entry, Descent and Surface landing | JPL EDL simulation developed by Bob Balaram, real-time and Monte Carlo capable |
| DSHELL | DARTS Shell |  |
| DSMS | Deep Space Mission Systems |  |
| DSN | Deep Space Network |  |
| DSN | Design |  |
| DSNE | Deep Space Network Emulator |  |
| DSIT | Data System Integration Team |  |
| DSOC | Deep Space Operations Center | DSN Data Flow |
| DSOT | Data Systems Operations Team |  |
| DSPA | Descent Stage Power Assembly |  |
| DSS | Deep Space Station | DSN Data Flow |
| DTE | Direct to Earth | Telecom related |
| DTE | Differential Thermal Expansion |  |
| DTM | Development Test Model or Dynamic Test Model |  |
| DTT | Downlink, Tracking, Telemetry | DSN Data Flow |
| DUHF | Descent UHF antenna |  |
| DUT | Delta UTC | Difference between TDT and UTC frames (DUT = TDT-UTC) |
| DV | Data Volume |  |
| DVS | Doppler Velocity Sensor |  |
| DVT | Data Validity Time | Roughly correlates to spacecraft clock (SCLK) time a data product was created by the rover compute element (RCE). |
| DVT | Dispensers, Volume, and Tube Storage |  |
| DW | Double Word (32-bits) |  |
| DWTS | Descent Stage Waveguide Transfer Switch |  |
|  | E |  |
| EAR | Export Administration Regulation |  |
| Earth-in-Control | Earth-in-Control Mode | See Spacecraft Modes within this list |
| EAS | Engineering Analysis Subsystem |  |
| EBCR | Engineering/Budget Change Request | Aka "ECR+Lien" |
| EBOX | Electronics Box | Part of SuperCam |
| ECA | Export Certified Author |  |
| ECAM | Engineering Camera |  |
| ECC | Error Correcting Code |  |
| ECR | Engineering Change Request |  |
| ECTH | EDU Corer Testbed Heavy |  |
| ED | Education |  |
| EDA | EDL Data Analysis | DSN Data Flow |
| EDAC | Error Detection And Correction | Functionality built into some memory chips |
| EDL | Entry, Descent, and Landing |  |
| EDLCAM | Entry, Descent, and Landing Cameras | encompasses, PUC, DDC, RUC, RDC (see other acronyms) |
| EDMG | Engineering Data Management Group |  |
| EDR | Export Document Reviewer |  |
| EDR | Experiment Data Record | i.e., raw data |
| EDT | Environmental Development Testbed | Testbed for coring in Mars environment. Located in building 248 10-ft chamber. |
| EDU | Engineering Development Unit |  |
| EECAM | Enhanced Engineering Camera | Synonymous with ECAM, this refers to the collection of the Hazcams, Navcams and Cachecam |
| EEE | Electrical, Electronic, and Electromechanical |  |
| EEIS | End to End Information Systems |  |
| EELV | Evolved Expendable Launch Vehicle | Altas 5 or Delta 4 series of LVs |
| EEV | Earth Entry Vehicle |  |
| EEPROM | Electrically Erasable PROM |  |
| EFPA | Entry Flight Path Angle |  |
| EG | Entry Guidance |  |
| EGSE | Electrical GSE |  |
| EH&A, EHA | Engineering, Health & Accountability | Channelized telemetry |
| EHASC | Engineering, Health & Accountability Selection Criteria |  |
| EI | Entry Interface |  |
| EICD | Electrical Interface Control Document |  |
| EIDP | End Item Data Package |  |
| EIM | Engineering Interface Model (MSL deliverable from the payloads) |  |
| EIP | Experiment Implementation Plan |  |
| EKO | Engineering Keepout |  |
| EKF | Extended Kalman Filter |  |
| EL | Elevation | can refer to both position in the sky but also to a motor |
| ELT | Electra Lite | UHF-A and UHF-B |
| EM | Engineering Models |  |
| EMC | ElectroMagnetic Compatibility |  |
| EMD | Earth product MetaData | Ground processed data product |
| EMF | ElectroMotive Force |  |
| EMI | ElectroMagnetic Interference |  |
| ENAV | Enhanced Autonav |  |
| EO | Engineering Operations |  |
| EOP | Experiment Operations Plan |  |
| EOP (GDSO) | End Of Pass | triggers notebooks |
| EOP (IDSO) | End Of Pipeline | EDR / FDR / RDR products have been delivered |
| EPDM | Engineering Product Data Management |  |
| EPGF | EDL Guidance Parameter Update (File) |  |
| EPO | Education and Public Outreach |  |
| EPS | Electrical Power System |  |
| EPU | EDL Parameter Update |  |
| ePDL | Engineering Payload Downlink Lead |  |
| ePUL | Engineering Payload Uplink Lead |  |
| ePUDL | Engineering Payload Uplink & Downlink Lead |  |
| EPW | Electrical Pulse Width |  |
| ERD | Environmental Requirements Document |  |
| ERE | Environmental Requirements Engineer |  |
| ERCO | European Relay Coordination Office |  |
| ERT | Earth Received Time | The UTC of an event received at a DSN station. |
| ESA | European Space Agency |  |
| ESCF | EHA Selection Criteria File |  |
| ETAS | Environmental Test Authorization and Summary |  |
| ETL | Export Technical Liaison |  |
| ETP | Environmental Test Program |  |
| ETPC | Entry Terminal Point Controller |  |
| ETRO | Estimated Time to Return to Operations | used by DSN for antenna station outage status and when we can expect the station to be online and available |
| ETU | Engineering Test Unit |  |
| EU | Engineering Unit | A high fidelity representation of the instrument with flight like interfaces and functionality |
| EUL | Engineering Uplink Lead | MSL EO role on uplink. See VSE for M2020 equivalent. |
| EUMB | Electrical Umbilical |  |
| EV | Entry Vehicle |  |
| EVM | Earned Value Management |  |
| EVR | EVent Report or Event Verification Record | Textual info/warning/error messages (telemetry) |
| E-WAM | Electronic Work Authorization Memorandum |  |
| EWAG | Educated Wild Guess |  |
| Expansion |  | auto-expanded sequences linked to specific activities |
|  | F |  |
| F/A | Fly Away segment |  |
| FA | Flight Acceptance |  |
| FAD | Formulation Authorization Document |  |
| FBM | Fault Behavior Manager |  |
| FBO | Federal Business Opportunities |  |
| FC | Flight Computer |  |
| FCS | Facility Contact Sensor | Limit Switch on Turret to detect contact |
| FDD | Functional Design Description |  |
| FDIR | Fault Detection, Isolation, Response |  |
| FDR | Fundamental Data Record |  |
| FDU | File Delivery Unit |  |
| FDU | Fault Detection Unit |  |
| FEI | File Exchange Interface |  |
| FFE | Friendly Front End |  |
| FFI | Forsvarets Forskning Institute | Developer of the RIMFAX instrument |
| FFRDC | Federally Funded Research and Development Center |  |
| FIT | First Installation Testing |  |
| FLASH |  | Modern version of EEPROM. Should be "flash" but sometimes written as "FLASH" |
| FLB | Forward-Link Binary | Equivalent to an SCMF, special type of compiled command product to be sent to the rover by relay through a Mars orbiter asset (i.e. MRO, TGO, etc.) |
| Flt | Flight |  |
| FLTS | Flight Like Test Set |  |
| FM | Flight Model OR Frame Manager |  |
| FMBB | Fluid Mechanic Bio Barrier | Contamination control method using a specially sized null space that limits fluid flow. Applicable to sample tube sealing in CSC. |
| FMPB | Fluid Mechanical Particle Barrier |  |
| FMECA | Failure Modes Effects Consequences Analysis |  |
| FOC | Field Oriented Control | Type of commutation for brushless motors |
| FOD | Foreign object debris |  |
| FOV | Field of View |  |
| FP | Fault Protection |  |
| FPA | Flight Path Angle |  |
| FPGA | Field Programmable Gate Array |  |
| FR | Flight Rule(s) |  |
| FR | Functional Requirements |  |
| FS | Flight System |  |
| FSE | Flight System Engineering |  |
| FSSE | Flight System Systems Engineering |  |
| FSW | Flight Software |  |
| FSWAVS | Flight Software/Avionics Subsystem |  |
| FSWTB | Flight Software Testbed |  |
| FT | Focus Technology |  |
| FTA | Fault Tree Analysis |  |
| FTA | First Time Activity |  |
| FTP | Focused Technology Program |  |
| FTS | Force Torque Sensor |  |
| FVS | File Verification Service |  |
| FWOB | forecasted weight on (drill) bit |  |
| FY | Fiscal Year |  |
|  | G |  |
| GCM | Global Circulation Model |  |
| GCM | Ground Current Monitor |  |
| GDRT | Gaseous Dust Removal Tool | captures the compressed gas component of the DRT |
| GDS | Ground Data System |  |
| GDSA | Ground Data System Analyst |  |
| GDSO | Ground Data System Operations | GDS Ops Org |
| GESTALT | Grid-based Estimation of Surface Traversability Applied to Local Terrain |  |
| GG | Gas Generator | Component of the Parachute Mortar Deployment System |
| GID | GNC Interface Drivers |  |
| GNC | Guidance Navigation and Control |  |
| GOTB | Gravity Off-load Testbed |  |
| GPB | General Purpose Board |  |
| GPCU | Guidance, Navigation, and Control Power Converter Unit |  |
| GPS | SAM Gas Processing System |  |
| Grnd | Ground |  |
| Grounded Plan |  | The plan after it has run through Copilot and which all of it's OBAs have been placed on the timeline. - Copilot is a feature of the COCPIT Tool - OBA pertains to Simple Planner - The PILOTs (an SO Role) will "ground the plan" (lock activities to the timeline) whether or not Simple Planner is used. |
| GSE | Ground Support Equipment |  |
| GST | Generic Sol Time |  |
| GTAG | GNC Technical Advisory Group |  |
| GUI | Graphic User Interface |  |
|  | H |  |
| H/S | Heatshield |  |
| HAFIQ | Hazard Avoidance Fault Image Queue |  |
| HazCam | Hazard Camera(s) (stereo pair) |  |
| HBB | Hardware Build Book |  |
| HBE | Helicopter Base Electronics |  |
| HBS | Helicopter Base Station | On the rover |
| HCD | Hardware Command Decoder |  |
| HDA | Hazard Detection and Avoidance |  |
| HELI | Mars Helicopter |  |
| HELO | Helicopter manager module in RCE flight software |  |
| HEMT | High Electron Mobility Transistor | DSN hardware |
| HES | Hall Effect Sensor |  |
| HESSTER | Human-Excel Shift Start Time Easily Readable |  |
| HGA | High Gain Antenna |  |
| HGAG | High Gain Antenna Gimbal (assembly) |  |
| HGAS | High Gain Antenna System |  |
| HIE | Helicopter Integration Engineer |  |
| HO | Helicopter Operarions |  |
| HOH | Heating over Handover |  |
| HPCU | Housekeeping Power Converter Unit |  |
| HRCR | Hardware Review and Certification Record |  |
| HRS | Heat Rejection and Recovery System |  |
| HS | Heatshield |  |
| HS | Humidity Sensor | Part of MEDA - mounted on RSM |
| HSA | Horizontal Swing Arm | Primary component of center differential pivot |
| HSS | High Speed Serial |  |
| HVMM | High Voltage Multiplier Module | PIXL module |
|  | I |  |
| I&T | Integration & Test |  |
| I/F | Interface |  |
| I/O | Input/Output |  |
| IAR | Instrument Accommodation Review |  |
| ICD | Interface Control Document |  |
| ICR | Implementation Completion Review | In relation to GDS development and implementation |
| ICU | Instrument Control Unit | Part of MEDA - mounted on RAMP |
| idio | idiosyncrasy |  |
| IDM | Interface Design, Mechanical |  |
| IDPH | Instrument Data Product Header |  |
| IDS | Instrument Data System | Development team |
| IDSA | Instrument Data System Archive | IDS archive team |
| IDSO | Instrument Data System Operations | IDS operations team |
| IE | Instrument Engineer |  |
| IFPA | Inertial Flight Path Angle |  |
| IG&N | Integrated Guidance and Navigation |  |
| IICD | Information Interface Control Document |  |
| ILC | Integrated Launch Cruise |  |
| IML | Instrument Manager Library |  |
| IMRCWG | International Mars Relay Coordination Working Group |  |
| IMU | Inertial Measurement Unit |  |
| IND | Interplanetary Network Directory |  |
| INCONS | Initial Conditions |  |
| INH | Inhibited |  |
| IO | Instrument Operations |  |
| IOC | Inorganic Contamination |  |
| IOM | Interoffice Memo |  |
| IPA | Integrated Pump Assembly |  |
| IPE | Integrated Processing and Execution |  |
| IPPR | Institutional Program Requirements |  |
| IR | Infrared |  |
| IRD | Instrument / Interface Requirements Document |  |
| ISA | Incident, Surprise, or Anomaly |  |
| ISEO | Integrated System Engineering Office |  |
| ISR | Implementation Start Review | In relation to GDS development and implementation |
| ITAR | International Traffic in Arms Regulations |  |
| ITL | Incompressible Test List |  |
| IV&V | Independent Verification and Validation |  |
| IVP | Inertial Vector Propagation |  |
|  | J |  |
| Jira | (not an acronym) | Bug-tracking system; "jira" or "zilla" is from Godzilla (the original language doesn't distinguish the sounds), so it's related to Bugzilla and Mozilla. |
| JPL | Jet Propulsion Laboratory |  |
| JSC | Johnson Space Center |  |
| JSON | JavaScript Object Notation |  |
|  | K |  |
| KIP | Key Information for Planning |  |
| KOZs | Keep Out Zones |  |
| KSC | Kennedy Space Center |  |
|  | L |  |
| L/C | Launch Cruise |  |
| L/D | Lift over Drag | Ratio of lift to drag |
| LAE | Launch Approval Engineering |  |
| LAMP | LAser MaPper | Laser instrument that illuminates target areas with laser and analyzes returns to determine position, orientation, range, etc. Being applied to MSR sample capture (with retroreflectors on target) and MSL hazard detection and avoidance sensing. |
| LAP | Look Ahead Plan |  |
| LaRC | Langley Research Center |  |
| LAURA | Langley Aerothermodynamic Upwind Relaxation Algorithm | 3-D structured, finite-volume Navier-Stokes solver |
| LBA | Launch Bit Assembly | Part of SCS. Abrading bit assembly engaged in Drill Chuck during launch. |
| LCAE | Launch/Cruise/Approach/EDL |  |
| LCC | Load Control Card |  |
| LCE | Launch Complex Equipment |  |
| LCO | Link Control Operator | DSN |
| LD | Launch Date |  |
| LGA | Low Gain Antenna |  |
| LIBS | Laser-Induced Breakdown Spectrometer | SuperCam subsystem |
| LIDAR | Light Detection And Ranging |  |
| Li-Ion | Lithium Ion |  |
| LMST | Local Mean Solar Time |  |
| LNUT | Last Nominal Uplink Time |  |
| LOE | Level of Effort |  |
| LOS | Line Of Sight | Boresight of cameras, lasers etc |
| LPCU | LCC and RCC Power Converter Unit |  |
| LPF | Low Pass Filter |  |
| LPHTA | Low Power High Torque Actuator |  |
| LPS | Laser Power Supply | For SHERLOC |
| LSA | Lightning Suppression Assembly |  |
| LSC | Launch Service Contractor |  |
| LSP | Launch Services Program |  |
| LSRD | Launch Systems Requirements Document |  |
| LST | Mars Local Time (MER Term) |  |
| LSTO | Launch Service Task Order |  |
| LTC | Low Thrust Characterization |  |
| LTST | Local True Solar Time | Assumes Sun highest in the sky at noon |
| LVS | Lander Vision System |  |
| LVDS | Low Voltage Differential Signaling |  |
| LVDT | Linear Voltage Distance Transducer |  |
| LVLH | Local Vertical, Local Horizontal | coordinate frame |
|  | M |  |
| M&M | Mast & Motion |  |
| M&P | Material and Processes |  |
| MA | Mission Assurance |  |
| MAC | Mass Acceleration Curve |  |
| Maestro | Rover Operations Control Software |  |
| MAHLI | Mars Hand Lens Imager | Turret-mounted high resolution camera |
| MAM | Mission Assurance Manager |  |
| MAP | Mission Assurance Plan |  |
| MaROS | Mars Relay Operations Service | Service used to deliver forward-link products to Mars Relay Network assets. |
| MAV | Mars Ascent Vehicle |  |
| MarsGRAM | Mars Global Atmospheric Model |  |
| MarsTerm |  | Terminal descent simulation? |
| Mastcam-Z | Mast Camera(s) (stereo pair, with zoom lenses) | Mast-mounted multispectral, stereoscopic imaging instrument developed by ASU & MSSS |
| MCA | Motor Control Assembly |  |
| MCB | Motor Controller Backplane |  |
| MCB-B | Motor Controller Break Board | Also called RMCB-B |
| MCCB | Mission Change Control Board |  |
| MCD | Motor Control Driver (Board) |  |
| MCE | Motor Control Electronics |  |
| MCFSW | Motor Control Flight Software |  |
| MCI | Mars Centered Inertial | Coordinate frame |
| MCIC | Motor Control and Interface Card |  |
| MCIF | Motor Control Interface Card |  |
| MCLK | Mission Clock |  |
| MCLN | Master Cleanup |  |
| MCMF | Mars Centered, Mars Fixed | Coordinate frame |
| MCPCU | Motor Control Power Converter Card |  |
| MCR | Mission Concept Review |  |
| MCS | Mission Control Subsystem |  |
| MCS | Motor Control Sensor (Board) |  |
| MCS | Motion Control Subsystem |  |
| MDC | Motor Drive Card |  |
| MDM | Micro-D Metal Connector |  |
| MDNAV | Mission Design and Navigation |  |
| MDP | Main Differential Pivot | Mobility pivots located on sides of rover. Sits between CDP and RDP. |
| MDS | Mission Data System | Software operating system/environment |
| MDS | Mortar Deployment System | Parachute Mortar Deployment System |
| MEB | SAM Main Electronics Box |  |
| MEDA | Mars Environmental Dynamics Analyzer | Rover-mounted instrument to characterize dust size and morphology and measure surface weather, developed by Instituto Nacional de Tecnica Aeroespacial (INTA) in Spain |
| MEDLI | Mars Entry Descent and Landing Instrumentation |  |
| MEL | Mass Equipment List |  |
| MEOP | Maximum Expected Operating Pressure |  |
| MEP | Mars Exploration Program |  |
| MEPAG | Mars Exploration Program Assessment Group |  |
| MER | Mars Exploration Rovers |  |
| Merge Set |  | - Multiple Activity Groups chained together into a single entity - Can only be created tactically using strategically-developed Activity Groups |
| MEX | Mars Express |  |
| MFSK | Multiple Frequency shift-keying | Type of tone emitted from SC during entry |
| MGA | Medium Gain Antenna |  |
| MGS | Mars Global Surveyor |  |
| MGSE | Mechanical GSE |  |
| MHDS | Mars Helicopter Delivery System |  |
| MHS | Mars Helicopter System |  |
| MI | Micro Imager |  |
| MICD | Mechanical Interface Control Document |  |
| MIDP | Mars Instrumentation Dev. Program |  |
| MIM | Mission Integration Manager | This is a KSC-related Launch role |
| MIM | Mission Interface Manager | This is a Deep Space Network-related role |
| MIMU | Miniature Inertial Measurement Unit |  |
| MIP | Mandatory Inspection Point |  |
| MIST | Mars Icy Sample Team |  |
| MIST | Mission Information System Testbed |  |
| MIUL | Material Identification and Usage Lists |  |
| MLA | Motor Load Assembly | Bank o' test motors |
| MLE | Mars Lander Engine | Modified throttleable Viking engine |
| MLI | Multi-Layer Insulation |  |
| MM | Mission Manager |  |
| MMAC | Modal Mass Acceleration Curve |  |
| MMM | Mastcam, MARDI, MAHLI | MSL acronym |
| MMPAT | Multi-Mission Power Analysis Tool | Power usage analysis tool |
| MMR | Monthly Management Review | (Also: Monthly Management Report) |
| MMRTG | Multi-Mission Radio-isotope Thermoelectric Generator |  |
| MO&DA | Mission Operations & Data Analysis |  |
| MOA | Memorandum of Agreement |  |
| Mob | Mobility |  |
| MOC | Mars Orbiter Camera |  |
| MOLA | Mars Observer Laser Altimeter | Instrument on Mars Global Surveyor used to generate topography map of Mars. Redefined reference geoid, mean ground level. |
| MOPs | Millions of Operations |  |
| MOS | Mission Operations System |  |
| MOSB | Multi-Operations Support Building | Facility at KSC |
| MOU | Memorandum of Understanding |  |
| MOT | MOTor control | MOT: MOTor control, pronounced like “moat (RCE FSW Motor control module which interfaces to higher-level modules for motion requests) |
| MOXIE | Mars Oxygen ISRU Experiment | Rover-mounted oxygen-producing In Situ Resource Utilization (ISRU) instrument developed by JPL & MIT |
| MPCS | Mission Data Processing and Control Subsystem | MSL |
| MPCU | MREU Power Converter Unit |  |
| MPF | Mars Pathfinder |  |
| MPPF | Multi-Payload Processing Facility |  |
| MPL | Mars Polar Lander |  |
| MPSET | Mars Program Systems Engineering Team |  |
| MR | Management Reserve |  |
| MREU | MSAP Remote Engineering Unit |  |
| MRO | Mars Reconnaissance Orbiter |  |
| MRR | Mission Risk Review |  |
| MRPP | NASA Mission Resilience and Protection Program | Formerly known as SAPP (Space Asset Protection Program) |
| MSA | Mission Support Area |  |
| MSAP | Multi-Mission System Architecture Platform |  |
| MSIA | MSAP Serial Interface Assembly |  |
| MSIADIS | MSIA Discretes Driver |  |
| MSEQ | MSLICE Sequencing | Service used for bulk compilation |
| MSL | Mars Science Laboratory |  |
| MSLICE | Mars Science Laboratory InterfaCE | An ensemble of software tools that provide the MSL mission with science data visualization and analysis, roundtrip data tracking, scheduling of integrated activity plans, modeling, simulation, and validation of activity plans built on the Eclipse RCP |
| MSM | Mars Surface Mission |  |
| MSM | Master/Submaster | generic term for the sequences the SIE generates or the tool that generates it |
| MSMaC | Master, Submaster, and Cleanup | unofficial term for the tool that generates SIE sequences (aka SSG which is the official APSS tool) |
| M/SM Mode | Master/Submaster Mode | See Spacecraft Modes within this list |
| Msn | Mission |  |
| MSPA | Multiple Spacecraft Per Antenna |  |
| MSPSP | Missile System Pre-launch Safety Plan |  |
| MSR | Mars Sample Return Mission |  |
| MSSS | Malin Space Science Systems | Developer of the Mastcam-Z instrument hardware |
| MSM | Mars Science Mission | Old name for Mars2020 mission |
| MSTB | Mission System Testbed |  |
| mstr | master sequence |  |
| MTAK | MPCS Test Automation Kit |  |
| MTIF | MSAP Telecommunications Interface Card |  |
| MTIFDIS | MTIF Discretes Driver |  |
| MTO | Mars Telecom Orbiter |  |
| MTP | Mars Technology Program |  |
| MTTMS | Mars Twenty-Twenty Mission Systems |  |
| MUA | Material Usage Agreement |  |
| MUT | Maximum Up Timer | Parameter that will reset the RCE into isolation mode if the FSW remains awake for too long. |
| MVN | Maven | orbiter around Mars |
| MVP | Minimum Viable Product | a sad state of affairs |
|  | N |  |
| NAIF | Navigation and Ancillary Information Facility |  |
| NASA | National Aeronautics and Space Administration |  |
| NavCam | Navigation Camera(s) (stereo pair) | For CEDL , refers to Optical Navigation Camera |
| NC | Normally Closed |  |
| NEA | Non Explosive Actuator |  |
| NEA | Nuclear Energy Agency |  |
| NEPA | National Environmental Policy Act |  |
| NLS | NASA Launch Services |  |
| NO | Normally Open |  |
| NOA | Network Operations Analyst |  |
| NOCT | Network Operations Control Team | DSN |
| NOPE | Network Operations Project Engineer | DSN |
| NPD | NASA Program/Project Management Directive |  |
| NPG | NASA Procedures abnd Guidelines |  |
| NPM | Non-Volatile Parameters Manager |  |
| NRC | National Research Council |  |
| NSI | Nuclear Strategic Initiative |  |
| NSI | NASA Standard Initiators |  |
| NSOE | Nominal Sequence of Events | To be used in absence of DKF |
| NVDS | Non-Volatile Data Storage |  |
| NVFS | Non-Volatile File System |  |
| NVM | Non-Volatile Memory |  |
| NVMCAM | Non-Volatile Memory and Camera Board |  |
| NVRAM | Non-Volatile RAM |  |
|  | O |  |
| OBA | Onboard Activity | - A Simple Planner container for a group of activities – equivalent to a Merge Set (or a single Activity Group that is not part of a Merge Set) - This is the base planning unit for Simple Planner, and is the level at which we can manipulate the plan tactically. - Simple Planner schedules at the OBA level, not the lower-level Activity level). - Like a “submaster” |
| Onboard Plan File |  | The file generated to send to the rover (replaces the Master sequence) |
| OBA | Optical Bench Assembly | For SHERLOC |
| OBOC | One-Bit-One-Core | (OBSOLETE) Sampling architecture in which tube and bit are sealed and stored together in the cache (one-time use bits) |
| OBOX | Optics Box | Part of SuperCam |
| OBP | Onboard Planner |  |
| OBP-in-Control | OBP-in-Control | See Spacecraft Modes within this list |
| OBS | Organization Breakdown Structure? |  |
| OBE | Overcome By Events |  |
| OCM | Organic Check Material | Blank rock analog material mounted on the rover that can be drilled and cached by SCS for sample calibration. |
| OCP | Over Current Protection |  |
| OD | Orbit Determination |  |
| OCS | Operational Cloud Store | File Management System that is an upgrade/replacement of the MSL ODS (Operations Data Server). Key Function is to Store, Save, and Retrieve Mission Operations Files. Metadata can be searched and read by team tools. Tools can automatically process data as it becomes available. |
| ODS | Operations Data Server | MSL's File Management System. On M2020, this is replaced by the OCS |
| ODY | Mars Odyssey |  |
| OFS | Optical Fiducial System | PIXL’s imaging system |
| OHR | Online Health Report |  |
| OIA | Operational Interface Agreement |  |
| OLR | Open Loop Receiver | DSN Data Flow |
| OMAM | Operations MIssion Assurance Manager |  |
| ONC | Optical Navigation Camera |  |
| OOCP | On Orbit Contingency Plan |  |
| OpNav | Optical Navigation | Optical imaging of targets such as moons of Mars to obtain better navigation knowledge |
| OPS | Operations |  |
| Oracle | JPL Project Acct. System |  |
| ORS | Operations Readiness Simulation |  |
| ORT | Operations Readiness Test |  |
| OSF | Overflight Summary File |  |
| OSHA | Occupational Safety and Health Administration |  |
| OSS | Operational Safety Survey |  |
| OT | Observation Table | MEDA term. Mobility also supports a different notion of Observation Tables (sequences called based on mobility or time triggers), but Mobility does not typically use the OT acronym. |
| OVP | Over Voltage Protection |  |
| OWLT | One-Way Light Time | The elapsed time it takes for light, or a radio signal, to reach a spacecraft or other body from Earth (or vice versa) |
|  | P |  |
| P/L | Payload |  |
| PAE | Pyro Arm Enable |  |
| PADS | Powder Acquisition Drill System |  |
| PAM | Power and Analog Module |  |
| PAMB | Power and Analog Module Backplane |  |
| Parcel |  | Strategically updated and V&V’d, version-controlled combination of the Activity Dictionary, Component Library, and Expansions (auto-expanded sequences linked to specific activities) |
| PARts | Parallel ATLO Rover test system |  |
| PATR | Phased Array Terrain Radar |  |
| PBC | Power Bus Control |  |
| PBDL | Primary Battery Depassivation Load |  |
| PBE | Predicted Best Estimate |  |
| PCA | Pressure Control Assembly |  |
| PCB | Payload Checkout Bench |  |
| PCC | Power Control Card |  |
| PCC | Parachute Closeout Cone |  |
| PCFG | Parameter Config File | A file that bulk changes parameters rather than sending a bunch of parameter setting commands. |
| PCI | Peripheral Component Interconnect | Data Bus |
| PCU | Power Conditioning Unit |  |
| PDC | Payload Downlink Coordinator |  |
| PDE | Propulsion Drive Electronics |  |
| PDI | Post Drive Imaging |  |
| PDL | Payload Downlink Lead |  |
| PDMS | Project Data Management System |  |
| PDP | Packetization of Data Products |  |
| PDPP | Post Downlink Product Processing |  |
| PDPR | ? Preliminary Design Peer Review ? |  |
| PDR | Preliminary Design Review |  |
| PDS | Parachute Decelerator System |  |
| PDS | Planetary Data System |  |
| PDSE | Project Data Systems Engineer |  |
| PDU | Power Distribution Unit |  |
| PDV | Powered Descent Vehicle | = descent stage stacked on the rover |
| PEDE | Planet Encircling Dust Events |  |
| PEF | Predicted Events File | SEQGEN output |
| PEL | Power Equipment List |  |
| PEL | Payload Engineering Lead | Supercam |
| PEM | Plastic Encapsulated Microcircuits |  |
| PEM | Project Element Manager |  |
| PFM | Proto-Flight Module |  |
| PFC | Pyro Firing Card |  |
| PFR | Problem Failure anomaly Reporting |  |
| PHSF | Payload Hazardous Servicing Facility |  |
| PI | Principal Investigator |  |
| PIA | Propellant Isolation Assembly |  |
| PID | Programmable Interface Discrete |  |
| PIE | Parameter Interface Etc |  |
| PILOT | Plan Integration Lead On Tactical | Person on console putting the activity plan together |
| PIP | Payload Information Package |  |
| PIWG | Principal Investigators Working Group |  |
| PIXL | Planetary Instrument for X-ray Lithochemistry | Turret-mounted instrument developed by JPL |
| PLA | Payload Adapter | Refers to launch vehicle payload adapter (not science payload) |
| Plan |  | The group of components that is being discussed in this planning interval The plan is viewed in the COCPIT Tool |
| PLC | Path Length Control |  |
| PLF | PayLoad Fairing | Launch vehicle payload fairing (not science payload) |
| PLGT | PFC, LCC, GID, and TMC | Pyro Firing Card (PFC), Load Control Card (LCC), Guidance Navigation and Control (GNC) Interface Driver (GID), and Telemetry Multiplexer (aka Mux) Card (TMC) |
| PMP | Payload Management Plan |  |
| PMP | Parts, Materials & Processes |  |
| PMSR | Project Mission Systems Review |  |
| PNG | Pulsed Neutron Generator (System) | DAN subsytem |
| PNTLIB | Pointing Control Library | M2020 FSW module used for Closed Loop Pointing |
| Pol | Polarizer |  |
| POR | Power-On/Reset |  |
| PORGs | Practical Operational Readiness Gambits | Training-only simulations |
| POSSE | Payload Operations Science Systems Engineer |  |
| POST | Program to Optimize Simulated Trajectories | High fidelity LaRC EDL simulation |
| PP | Planetary Protection |  |
| PPEL | Planetary Protection Equipment List |  |
| PPO | Planetary Protection Officer | NASA PP official |
| PPC750 | Power PC 750 |  |
| PPPCS | Pointing, Phase Position Coordinate System |  |
| PPPCS (3PCS) | Pointing, Positioning, Phasing, and Coordinate Systems Documents |  |
| PPS |  | Radiator related? |
| PQV | Packaging Qualification and Verification |  |
| PRA | Probablistic Risk Assessment |  |
| PRA | Project Resource Analyst |  |
| PRB | Pyro Relay Board |  |
| PRD | Project Requirements Document |  |
| PROM | Programmable Read-Only Memory |  |
| Prodapt | Proprioceptive Adaptive | Prodapt Drilling |
| PRT | Platinum Resistance Thermometer |  |
| PS | Proxy Science |  |
| PSA | Parts Stress Analysis |  |
| PSC | Power Switch Card |  |
| PSE | Project Systems Engineering |  |
| PSG | Project Science Group |  |
| PSIG | Project Science Integration Group |  |
| PSP | Pre-launch Safety Plan |  |
| PSS | Parachute Support Structure |  |
| PTCC | Percuss to Clean Chuck |  |
| PTI | Percuss To Ingest | In relation to sample processing. |
| PTY | Pseudo TTY –> Pseudo Teletype |  |
| Pub | Public |  |
| PUC | Parachute Uplook Camera | x3 |
| PUL | Payload Uplink Lead |  |
| PULM | Payload Uplink Lead Manager | Mastcam-Z, WATSON |
| PV | Pyro Valve |  |
| PWR | Power |  |
| PWTB | Power Thermal Battery |  |
| PYTB | Pyro Thermal Battery |  |
|  | Q |  |
| QA | Quality Assurance |  |
| QMDT | Qualification Model Dirty Testing | which is the mission |
| QMS | SAM Quadrupole Mass Spectrometer |  |
| QMR | Quarterly Management Review |  |
| QRM | Quantitative Relief Model | additional stereoscopic imaging with MAHLI/WATSON instruments |
| QTP | Qualification Test Program |  |
|  | R |  |
| R4R | Run For Record |  |
| R/RC | Removal / Re-certification Record | For when hardware is removed from ATLO for re-work and then is re-delivered to ATLO. |
| R/T | Realtime |  |
| RA | Robotic Arm |  |
| RAM | Random Access Memory |  |
| RAMP | Rover Avionics Mounting Plate |  |
| RAS | Robotic Arm Science |  |
| RAT | Rock Abrasion Tool |  |
| RBA | Regolith Bit Assembly | Part of SCS. Bit assembly used for regolith collection using the Drill. |
| RBAT | Rover Battery | Secondary |
| RBAU | Rover Battery Assembly Unit |  |
| RBCB | Rover Battery Control Board |  |
| RBF | Remove Before Flight | Covers |
| RBF | Rover Body Fixed | coordinate frame |
| RCA | Root Cause Analysis |  |
| RCC | Relay Control Card |  |
| RCE | Rover Compute Element | Major Avionics Assembly |
| RCS | Reaction Control System |  |
| RCT | Record Creation Time | For any given product, this is the time corresponding to when the first frame containing data from that product is stored (and available for query) on TDS. |
| RCXS | Rover Coaxial Transfer Switch |  |
| RDC | Rover Downlook Camera |  |
| RDP | Rocker Deploy Pivot | Mobility pivots located on sides of rover. Responsible for deployment of mobility system. |
| RDR | Reduced Data Record | i.e. calibrated, geolocated data |
| RDS | Radiation and Dust Sensor | Part of MEDA - mounted on rover deck |
| REMS | Rover Environmental Monitoring Station | Instrument |
| REU | Remote Electronics Unit |  |
| REXMIT | Re-transmit |  |
| RF | Radio Frequency |  |
| RFA | Request For Action |  |
| RFI | Request for Information or Radio Frequency Interference |  |
| RFP | Request for Proposal |  |
| RFS | Radio Frequency S/S |  |
| RGA | RSTA (Returnable Sample Tube Assembly) Glove Assembly | The physical thing being dropped into a sample cache. Also just referred to as "Tube". |
| RHU | Radio-isotope Heating Unit |  |
| RIMFAX | Radar Imager for Mars' subsurFAce eXperiment | Rover-mounted ground penetrating radar (GPR) instrument, developed by Forsvarets Forskning Institute (FFI) in Norway |
| RIMU | Rover IMU | LN200 |
| RIPA | Rover Integrated Pump Assembly | Fluid heat mover |
| RKSML | Rover Kinematic State Markup Langauge |  |
| RLG | Ring Laser Gyro |  |
| RLGA | Rover Low Gain Antenna | (LGA's get the R for Rover because witht the cruise and descent stages there is another LGA (the PLGA). There is also a TLGA (tilted) and a DLGA (descent stage) |
| RMCA | Rover Motor Controller Assembly | Major Avionics Assembly |
| RMI | Remote Microscopic Imager | on supercam |
| RML | Rover Markup Language |  |
| RMOTS | Rover Mission Operations Training System |  |
| RMS | Resource Management System |  |
| RO | Robotics Operations |  |
| ROAMS | Rover Analysis, Modeling and Simulation |  |
| ROASTT | Rover Operations Activities for Science Team Training |  |
| ROCS | Robot Operational and Computational Service |  |
| RODinT | Robotic Operations Downlink Teamtools | Team that helps manage subsystem teamtools related to RO downlink (e.g. mech_data_tools, ro_events). You may see "Managed by RODinT" in github repository About fields. |
| ROI | Region of Interest |  |
| ROLO | ROver LOcalization | Web-based geospatial tool for localizing the rover. Results are pushed to PLACES. This tool is only used by the Mapping Specialists. |
| ROM | Rough Order of Magnitude |  |
| ROM | Read-Only Memory |  |
| ROP | Rate of penetration (for drilling) |  |
| RPA | Rover Power Assembly | Major Avionics Assembly |
| RPAB | Rover Power Assembly Backplane |  |
| RPAM | Rover Power and Analog Module | Major Avionics Assembly |
| RPFA | Rover Pyrotechnic Firing Assembly |  |
| RPM | Revolutions Per Minute |  |
| RPM | Resonant Percussion Mechanism | Corer mechanism |
| RPRA | Rover Pyro Relay Assembly | Major Avionics Assembly |
| RPS | Radio-isotope Power Source/System |  |
| RPS | Rover Planning Subsystem |  |
| RPT | Report |  |
| RSB | Remote Serial Bus |  |
| RSCS | Rover Sample Cache System |  |
| RSD | Rover Shunt Driver |  |
| RSM | Remote Sensing Mast |  |
| RSR | Rover Shunt Radiator |  |
| RSVP | Robot Sequencing and Visualization Platform |  |
| RTCIU | Remote Terminal Communications Interface Unit |  |
| RTDP | real time data product | Used during EDL |
| RTG | Radio-isotope Thermoelectric Generator |  |
| RTI | Radar Target Identification |  |
| RTI | Real Time Interrupt |  |
| RTLT | Round-Trip Light Time |  |
| RTOS | Real Time Operation System |  |
| RTSA | Returnable Sample Tube Assembly |  |
| RTT | Round Trip Tracking |  |
| RUC | Rover Uplook Camera |  |
| RUHF | Rover UHF antenna |  |
| RVR | Rover |  |
| RWEB | Remote Warm Electronics Box | Enclosure housing the SuperCam Mast Unit on top of the remote sensing mast |
|  | S |  |
| S/C | Spacecraft |  |
| S/S | Subsystems |  |
| SA | Sample Acquisition |  |
| SA | Solar Array |  |
| SABS | Solar Array Battery Simulator |  |
| SAE | Sample Acquisition Engineer |  |
| SAF | Spacecraft Assembly Facility |  |
| SAF | Station Allocation File |  |
| SAPP | Surface Attitude Position and Pointing |  |
| SAR | Synthetic Aperture Radar |  |
| SAR | Science Activity Request |  |
| SARB | String Arbitration |  |
| SAS | Solar Array Simulator |  |
| SASF/SATF | Spacecraft Activity Sequence File/Spacecraft Activity Type File |  |
| SASPaH | Sample Acquisition & Sample Processing and Handling |  |
| SBA | SHERLOC Body Assembly |  |
| SBE | Single Bit Error |  |
| SBP | SuBsonic Parachute |  |
| SCAM | SuperCam |  |
| SCCT | SuperCam Calibration Target |  |
| SCD | Source Control Drawing |  |
| SCET | Spacecraft Event Time | Spacecraft Event Time is the UTC time onboard the spacecraft. It is equal to TRM + OWLT. ERT is equal to SCET + OWLT |
| SCILO | SCIence LOcalization | Web-based tool for generating image footprints and orbital XYZ data (XOZ) RDR products based on NAVCAM, HAZCAM, ZCAM, and SCAM images projected onto the orbital DEM. This tool is only used by the Mapping Specialists. |
| SCLK | Spacecraft Clock | Spacecraft Clock is the value of a counter onboard a spacecraft. SCLK has a nearly-direct relationship with SCET: it is the best possible on-board estimate of SCET. SCLK is not as constant and stable as the UTC-derived SCET. |
| SCMF | Spacecraft Command Message File | Standard compiled command product with information unique to each spacecraft |
| SCMU | SuperCam Mast Unit |  |
| SCS | Sampling and Caching Subsystem | Subsystem to sample and store Mars rock samples for future return to Earth. |
| SDE | Scanner Demodulator Electronics | For SHERLOC |
| SDF | Science Data Frame |  |
| SDRAM | synchronized dynamic random access memory |  |
| SDRL | Subcontract Data Requirements List |  |
| SDST | Small Deep Space Transponder |  |
| SDT | State Database Tool |  |
| SDT | Science Definition Team |  |
| SE | Spectrometer Electronics | For SHERLOC |
| SE | System Engineering |  |
| SECC | Second Chance (for EDL) |  |
| SECR | Support Equipment Certification Record |  |
| SEE | Single Event Effects Analysis |  |
| SEFI | Single Event Functional Interrupt |  |
| SEL | Science Engineering Liason | Operations role. |
| SEP | Solar Electric Propulsion | Pyro driven separation nuts |
| SEP | Sun-Earth-Probe | angle used during conjunction |
| SEQER | Sequence Checker |  |
| SEU | Single Event Upset |  |
| SEWAR | Support Equipment Work Authorization Request |  |
| SFDU | Standard Format Data Unit | This is the CCSDS-defined unit of data stored in TDS (or more specifically, SFDU Transfer Frames). |
| SFP | System Fault Protection |  |
| SGNC | Surface Guidance, Navigation, and Control |  |
| SHA | Sampling Handling Arm | Within the ACA (Adaptive Caching Assembly) |
| SHERLOC | Scanning Habitable Environments with Raman & Luminescence for Organics & Chemicals | Turret-mounted instrument developed by JPL |
| SIA | System Interface Assembly |  |
| SIP | simulation input profile | Input file for MMPAT, usually not capitalized |
| SID | Switching Interface Device/Drivers | Required for switching with REU |
| SIE | Sequence Integration Engineer |  |
| SIO | System Input Output |  |
| SIPS | Science Instrument Purge System |  |
| SIR | System Integration Review (aka ATLO Readiness Review) |  |
| SIRCA | Silicon Impregnated Refractory Ceramic Ablator | TPS material |
| SIS | Software Interface Specification |  |
| SIT | System Integration and Test |  |
| SITL | Software In The Loop |  |
| SLGA | Switched Low Gain Antenna |  |
| SMM | Spacecraft Mode Manager |  |
| SMP | Software Management Plan |  |
| SMSA | Surface Mission Support Area |  |
| SN | Serial Number |  |
| SNC | Sampling 'n' Caching |  |
| SNC | Mars Meteorite (Shergotty-Nahkla-Chassigny) |  |
| SNET | Sequence No Earlier Than |  |
| SNR | Signal-to-Noise Ratio |  |
| SO | Science Operations |  |
| SOAS | Science Operations Analysis Subsystem |  |
| SOCER | SSim Output Checker | Also see SSim |
| SOPS | Surface Operations Venue |  |
| SORT |  | JSC CEDL simulation tool |
| SOWG | Science Operations Working Group |  |
| SOX | Surface Operations Transition |  |
| SPA | Spindle / Percussion Assembly |  |
| SP3T | Single Pole 3 Throw Switch |  |
| SP4T | Single Pole 4 Throw Switch |  |
| Spacecraft Modes | Spacecraft Modes | There are three spacecraft control regimes - Earth-in-Control mode (aka Master/Submaster mode) - OBP-In-Control (aka Simple Planner / OnBoard Planner mode) - Autonomous (automode) -a very simple set of rules are in control of when the rover wakes up and shuts down. The rover wakes up for comm windows and goes back to sleep when the comm windows are over |
| SPAD | Sample Preparation And Distribution | Spring '02 study commissioned by MPSET |
| SPADE | Sample Preparation and Distribution Experiment | Focused technology task |
| SPaH | Sample Preparation and Handling |  |
| SPAM | Sample Processing and Acquisition Manager |  |
| SPG | Single Point Ground |  |
| SPICE | Spacecraft, Planet, Instrument, C-matrix, Event (Software) |  |
| SPO | Schedule Process Owner | SPO office governs the scheduling process, as well as network downtimes, maintenance, deadlines |
| SPOS | Spacecraft Power Operations Suite | Power Ops subsystem tools and scripts |
| SPS | Service Preparation Subsystem | For the DSN |
| sPUL | Science Payload Uplink Lead |  |
| SQA | Software Quality Assurance |  |
| SRAM | Static RAM |  |
| SRAP | Slip Risk Assessment Process |  |
| SRCR | Software Review and Certification Record |  |
| SRG | Sterling Radio-isotope Generator |  |
| SRLC | SHERLOC | A shortened abbreviation for the SHERLOC instrument. |
| SRM | Shared REU Memory |  |
| SRM | Solid Rocket Motor |  |
| SRR | System Readiness Review |  |
| SRS | Shunt Radiator Simulator |  |
| SRT | Surface Removal Tool |  |
| SS | Surface System |  |
| SSA | Star Scanner Assembly |  |
| SSE | Simulation and Support Equipment |  |
| SSE | Sensor Support Electronics | MEDLI2 Power/Data Electronics Box |
| SSE | Spacecraft Systems Engineer | MER TDL on uplink shift, particularly once the TDL absorbed most other engineering roles. See VSE for M2020 equivalent |
| SSG | SIE Sequence Generator | interprets the cocpit plan into master, submaster, and cleanup sequences (aka M/SM, or MSMaC) |
| SSim | Surface Simulation | Flight-sofware-in-the loop simulation on Linux for surface operations |
| SSO | Single Sign On |  |
| SSO | Strategic Sampling Operations |  |
| SSF | Spacecraft Sequence File | A legacy SEQGen format that is an intermediate format used in compilation |
| SSP | SuperSonic Parachute |  |
| SSPA | Solid State Power Amplifier | Telecom related |
| SSPT | Sorting, sieving, portioning, and (sample) transfer |  |
| SSE | Scanner Servo Electronics | For SHERLOC |
| SSS | Surface Sampling System |  |
| SSTAGE | Surface Staging Area | GDSO |
| SSWG | Surface Scenario Working Group |  |
| STA | SHERLOC Turret Assembly |  |
| STB | System Testbed |  |
| STB | State Table Broadcast |  |
| STIG | Spindle Twin-Input Gearbox |  |
| STM | Structural Test Model |  |
| STM | Spacecraft Time Message |  |
| STOP | Sample Threshold Observation Protocol | *original definition |
| STOP | Standardized Observation Protocol | *current definition (4/7/26) |
| STR | Special Test Request |  |
| STR | Strategic Traverse Route |  |
| STSA | Sample Tube Storage Assembly |  |
| STV | System Thermal Vacuum |  |
| SuperCam |  | Mast-mounted instrument developed by Los Alamos National Labs and CNES |
| SUROM | Start-Up ROM |  |
| SW | Software |  |
| SWA | Subcontract Work Assignment? |  |
| SWG | Software Guidance |  |
|  | T |  |
| T/R | Transceiver | Radar related electronics module |
| T/W | Thrust to Weight ratio |  |
| TAYFE | Test As You Fly Exception | Exceptions to V&V |
| TB | Testbed |  |
| TBAT | Thermal BATtery | Primary |
| TBD | To Be Determined |  |
| TBLGA | Tilted Backshell Low Gain Antenna |  |
| TBR | To Be Revised |  |
| TC | Telemetry Collection |  |
| TCA | Thruster Cannister Assembly |  |
| TCC | Transmission Control Criteria | Every data product has a Transmission Control Criteria (TCC) attribute controlled as a parameter... A data product will be transmitted on a particular communications pass (either controlled by CBM or when packetization is otherwise enabled) if any of its TCC criteria match, through a bitmask, the Communication Path of the window. This capability allows particular data products to be transmitted only on particular types of communications windows. |
| TCM | Trajectory Correction Maneuver | Performed during cruise phase |
| TCRE | Thermal Cycle Resistant Electronics |  |
| TCS | Thermal Control System |  |
| TCXO | Temperature Control Crystal Oscillator |  |
| TD | Touchdown |  |
| TDAS | Thermal Data Acquisition System |  |
| TDL | Tactical Downlink Lead | Operations role that is responsible for overseeing downlink analysis and leading the downlink process for tactical operations. Similar to the Vehicle Systems Engineer (VSE) but with a more narrow, downlink focus. |
| TDR | Tile Data Record |  |
| TDS | Telemetry Data Server | DSN Data Flow |
| TDS | Terminal Descent Sensor |  |
| TDT | Touchdown Dynamics Testbed |  |
| TEC | Thermo-Electric Cooler/Cooling |  |
| TEICA | Thermal Energy Indication and Coring Analysis | Manufactured rock that will phase change above a certain temp reached during drilling |
| TES | Thermal Emission Spectrometer |  |
| TF | Transfer Frame |  |
| TFP | Telecom Forecaster Predictor |  |
| THCL | cruise THruster CLuster |  |
| TiB | Tube-in-Bit | Sampling architecture in which tube is removed from a reusable bit, sealed, and stored in the cache (reusable bits) |
| TID | Total Ionizing Dose |  |
| TIL | Tactical Implementation Lead | MER/MSL equivalent = TUL |
| TIM | Technology Interchange Meeting |  |
| TIRS | Thermal Infrared Sensor | Part of MEDA - mounted on RSM |
| TLGA | Tilted Low Gain Antenna |  |
| TLM | Telemetry |  |
| TM | Telemetry |  |
| TMC | Telemetry Multiplexer Card | aka Mux |
| TMRV | Touchdown and Mobility Research Vehicle |  |
| TMT | Tube Manipulation Testbed |  |
| TMV | Triple Majority Vote | PWR uses it in the hardware (ask Loren Jones) |
| TPS | Thermal Protection System |  |
| TPS | Third Party Software |  |
| TRCR | Technology Review and Certification Record |  |
| TRL | Technology Readiness Level |  |
| TRM | Transmit/Receive Module | The UTC time of uplink from Earth. |
| TRM | Transmission time |  |
| TRN | Terrain Relative Navigation |  |
| TRR | Test Readiness Review |  |
| TRT | Temperature Requirement Table |  |
| TSL | Tactical Science Lead |  |
| TSS | Tactical Science Support |  |
| TT | Thread Test |  |
| TTACS | Test Telemetry and Command System |  |
| TTC&DM | Tracking, Telemetry, Command & Data Management |  |
| TCP | Tool Control Point |  |
| TTCP | Technology Transfer Control Plan |  |
| TTE | Turn-to-Entry |  |
| TU | Testbed Unit | A medium fidelity representation of the instrument with flight-like interfaces and representative limited functionality, to be integrated into a JPL testbed. |
| TU | Timing Unit |  |
| TUL | Tactical Uplink Lead | Heritage title from MER/MSL, see TIL for M2020. |
| TVA | Throttle Valve Assembly |  |
| TWTA | Traveling Wave Tube Amplifier | Telecom related |
| TXL | Tube Exchange Lock | ACA subsystem |
| TZ | T-Zero |  |
|  | U |  |
| U/L | Uplink |  |
| UART | Universal Asychronous Receiver Transmitter |  |
| UAI | Use As Is |  |
| UAT | User Acceptance Test |  |
| UDA | Up/Down Converter Assy |  |
| UHF | Ultra High Frequency |  |
| UHP | Ultra High Purity | Often used to describe quality of gas for purging |
| UMR | Uplink Management Report | List of sequence being sent to the rover, being executed on tosol. |
| UMT | Upload Metadata Table | Summary of all the metadata on the sequences to be uplinked to the spacecraft |
| USGS | United States Geological Survey |  |
| USO | Ultra-Stable Oscillator |  |
| UT | Universal Time | Universal Time also called Zulu (Z) time, was previously called Greenwich Mean Time, GMT. It is common to see outdated references to GMT, even in currently operating flight projects. It is also common to encounter references to UT or GMT when the system actually in use is UTC, for example, "Uplink the command at 1801Z." |
| UTC | Coordinated Universal Time | The world-wide scientific standard of timekeeping. Its reference point is Greenwich, England |
| UV | Ultra Violet |  |
|  | V |  |
| V&V | Validation and Verification |  |
| V5 | Virtex-5 | FPGA in VCE |
| VA | Verification Activity |  |
| VAG | Verification Activity Group |  |
| VALT | Vibrating Actuator Life Test | GSE for placing corer actuators in vibration environment |
| VAP | Verification Activity Plan |  |
| VAMP | Vertical Avionics Mounting Plate | Vertical plate inside Warm Electronics Bay |
| VC | Virtual Channel | DSN Data Flow |
| VCE | Vision Compute Element |  |
| VCFC | Virtual Channel Frame Count | see VC above |
| VDE | Valve Drive Electronics |  |
| VDP | Virtual Data Product | Used to index data stored in Instrument NVRAM |
| VeST | Vehicle System Team | The combined VSE and TDL roles |
| VHP | Vapor Hydrogen Peroxide | Sterilization Methodology |
| VI | Verification Item |  |
| VISTA | Visualization for Telemetry Analysis |  |
| VL1 | Viking Lander 1 |  |
| VL2 | Viking Lander 2 |  |
| VLBI | Very Long Baseline Interferometry |  |
| VM | Virtual Machine | Type of Sun workstations used in MSL |
| VO | Visual Odometry |  |
| VOCA | Voice Operational Communications Assembly |  |
| VRE | Venue Reliability Engineer | GDSO role |
| VSA | Vertical Swing Arm | Primary component of Main Differential Pivots |
| VSE | Vehicle Systems Engineer | Operations role that is responsible for systems-level health and safety of the rover. Similar but more broad than the Tactical Downlink Lead (TDL) role. MSL role = EUL, MER = SSE |
| VSTB | Vehicle System Testbed |  |
| VTT | Visual Target Tracking |  |
|  | W |  |
| W/B | Workbench |  |
| WA | Work Assignment (or Agreement?) |  |
| WAG | Wild Guess | Used in reference to low confidence estimations |
| WAG | Waiver Approval Group |  |
| WART | WSTS Automated Regression Testing |  |
| WATER | Wrist and Turret Actuator |  |
| WATSON | Wide-angle Tool for Spacecraft Observations and iNvestigations | Proposed turret-mounted engineering and facilities camera hosted by the SHERLOC instrument |
| WBS | Work Breakdown Structure |  |
| WCA | Worst Case Analysis |  |
| WCC | Worst Case Cold |  |
| WCH | Worst Case Hot |  |
| WEB | Warm Electronics Box |  |
| WG | Working Group |  |
| WOB | weight on (drill) bit |  |
| WP | Work Package |  |
| WS1 | Wind Sensor 1 | Part of MEDA - mounted on RSM, not deployable |
| WS2 | Wind Sensor 2 | Part of MEDA - mounted on RSM, deployable |
| WSA | Wheel and Steering Actuator | Heritage actuator made by Cobham (formerly Aeroflex), used for drive and steer on the rover. |
| WSTS | Work Station Test Set |  |
| WTA | Witness Tube Assembly |  |
| WTS | Waveguide Transfer Switch |  |
|  | X |  |
| X-band | Satellite (microwave) frequency, as opposed to UHF DTE |  |
| XDR | eXtracted Data Record | Supercam raw data upstream of EDR |
| xMCA | (R or D) Motor Control Assembly | Refers to the common aspects of the RMCA and DMCA |
| XRD | X-Ray Diffraction |  |
| XRF | X-Ray Fluorescence |  |
|  | Y |  |
|  | Z |  |
| ZAR | Zone Access Request | for DOM |
| ZCAM | MastCam-Z Camera |  |
| ZDM | Mastcam-Z Data Manager |  |

# Simple Planner Acronyms & Glossary

| Term | Acronym | Definition |
| --- | --- | --- |
|  | A |  |
| Abort |  | OBP terminates an ongoing OBA. If the OBA has a sequence running and the "deactivate_on_abort" flag is set to TRUE for the OBA, aborting an OBA results in its sequence being terminated. If the OBA has a cleanup sequence specified, the cleanup sequence will be activated on abort. The exception to this is when a safing event deactivates an ongoing plan, in which case the cleanup sequences will not run and OBP will rely on the safing response to place the rover into a safe state. Examples of OBA abortion: - A generic OBA is aborted when reaching its cutoff time minus its cleanup duration, in order to ensure that the activity is fully terminated prior to the cutoff time. - Preheat OBAs are aborted when their users are vetoed. This is not a nominal case. - Maintenance heating OBAs are aborted when their users are no longer active. This is the nominal case for ending a maintenance heating OBA. - All ongoing OBAs are aborted upon plan deactivation. - In the event that a zero duty cycle maintenance heating OBA runs long into a time of day where heating would start duty cycling, and the increase in power usage is modeled to cause a peak power violation, active OBAs will be aborted until the peak power limit is satisfied. |
| Activity |  | The COCPIT planning unit that is used to model resources (power, data volume, duration, claims and parallelism) as well as generate sequences with auto-expansion. Activities are strategically defined in the Activity Dictionary as part of the parcel. The FSW construct "Onboard Activity" is also colloquially referred to as an activity. See "Onboard Activity" for details. |
| Activity Group | AG | A COCPIT container for a group of activities that occur sequentially and/or in parallel with each other. Developed strategically as a part of the parcel. |
| Activity Hopper |  | When OBP generates a schedule, it limits the number of activities it considers for scheduling to the number specified by the max_considered PLAN parameter. If the OBPF contains more non-communication window OBAs than max_considered, then the extra OBAs are said to be in the Activity Hopper. The activities that plan considers during scheduling constitute the Considered Set. Note that comm windows and committed activities, are not scheduled and, therefore, do not count toward the Considered Set. As time progresses, activities will be executed. When PLAN re-schedules, it does not have to consider these already-executed activities and, as a result, activities move from the Activity Hopper into the Considered Set. The overall behavior allows operations to include a large set of OBAs in an OBPF while constraining the amount of work PLAN has to do when scheduling with the max_considered parameter. |
| Automode |  | Mode of shutdown control in which FSW decides when to shutdown and wake back up. When there is no master sequence or OBP plan file running, the rover is in automode. Note that being in automode does not mean that a safing event has occurred, but a safing event will place the spacecraft into automode. Once a master sequence finishes running through all runout activities without successfully handing over, the rover will drop into automode. If OBP fails to handover to both the next plan file and the runout plan file, the rover will drop into automode. In automode, the rover will wake up for and execute each comm window onboard. If the rover has not safed, the nominal comm windows will be honored, otherwise the rover will execute safing windows. |
| Autonomous Activity |  | An OBA that is generated autonomously by FSW. These are not specified in the plan file. This includes preheat, maintenance heating, wakeup, and shutdown activities. |
|  | B |  |
| Bridging OBA |  | A Bridging OBA will - Force the rover stay awake between backbones. It will not allow the rover to nap. - Ensure heating is maintained across multiple OBAs without OBP creating separate preheat/maintenance heating OBAs for each. Example: - Example See Bridge OBA used on Sol 882 in-between Sample Acquisition and Sample Processing backbones to heat sealing station and avoid a nap. Who writes the bridging OBA: - An Operator (PILOT, OBP Chair, etc.) whoever is delivering the sequence that needs the bridging OBA - Bridging OBAs do not get generated automatically by Copilot or OBP. |
| Brittle |  | Describes a plan being constrained tightly such that the likelihood of activities being dropped from the schedule is increased. For example, pinning the start time for a heated OBA would be a brittle constraint, as a preheat taking longer than expected to reach target temperature would result in the OBA failing to start and being dropped from the schedule. |
|  | C |  |
| Cleanup (noun) |  | Refers to an OBA's cleanup sequence. A cleanup sequence may be specified for a generic OBA, which is used to ensure that the activity is completely cleaned up before ending the OBA. In SP mode, a cleanup sequence is only activated in the event that its OBA is aborted, unlike MSM mode where cleanup sequences run unconditionally. This means that, in SP mode, the OBA sequences must be able to fully clean up the activity upon completion and not rely on the cleanup (e.g. an RSM stow in a submaster). |
| Commit |  | When an OBA has become committed, the PLAN task has handed over authority of when an OBA starts to the Plan Controller task. In other words, the OBA may start executing once its eligibility requirements have been met (execution time range, CNF, etc.). Generally, OBAs become committed when entering the scheduling window. For heating activities, the preheat and maintenance heating do not commit until the preheat actually starts, which can vary based on temperature monitoring. The heated activity (i.e. the heating user) becomes committed when either the preheat starts or the preheat's scheduled start enters the scheduling window. |
| Component |  | A group of one or more Activity Groups meant to satisfy some intent. Defined and strategically managed/V&V’d in the Component Library. |
| Component-based Campaign Planning, Implementation, and Tactical | COCPIT | FIXME |
| Considered Set |  | The set of activities that PLAN considers when scheduling and defined by the max_considered PLAN parameter. See Activity Hopper for more details. |
| Constraint |  | Rules, specified in COCPIT (and the AD), which communicate to OBP when an activity can be scheduled. These are added to OBAs in the plan file to constrain timing and/or ordering. |
| Conjunctive Normal Form | CNF | In layman's terms, this is an AND statement of OR statements that is used to communicate dependency constraints to OBP in the plan file. Each generic OBA may have a CNF specified. A valid example of a CNF could be as follows: - A is in progress AND ( B completed with success OR C completed with success ) AND (D is finished OR E is finished) AND F is not scheduled Invalid CNF examples (which could be converted into conjunctive normal form using boolean algebra): - A is finished OR B is in progress OR ( C completed with success AND D completed with success ) - A is finished AND ( B completed with success OR ( C is in progress AND D is in progress ) ) |
| Copilot |  | Capability within COCPIT that generates a schedule of activity groups (OBAs), pre-heats and maintenance heating, CPU activities that satisfy all constraints. The scheduling algorithm is based on the FSW Onboard Planner algorithm. - Copilot is used in both Master/Submaster planning mode (current paradigm) and in Simple Planner planning mode to help the planning team understand what may happen on the rover based on plan contents and constraints |
|  | D |  |
| Dominant Node |  | FIXME |
| Drop |  | An activity that was previously placed in the schedule onboard is no longer being included in the schedule (e.g. an activity will drop from the schedule if an activity it depends on completing with success is aborted). |
|  | E |  |
|  | F |  |
| Fragmentation |  | Refers to the behavior of OBP dropping and/or reordering OBAs unnecessarily. |
|  | G |  |
| Gatekeeper |  | A synthetic OBA that is added to the plan file for heated OBAs that use the FINISHED_OR_NOT_SCHEDULED dependency in COCPIT. Mitigates fragmentation and the risk of an entire dependency chain of activities being dropped from the schedule. |
| Generic (noun) |  | A ground-specified OBA included in the plan file to activate a sequence (e.g. EKO, submaster, RP backbone, etc.) or act as a modeling placeholder. A generic OBA that specifies a sequence will activate the sequence when the OBA starts, and the OBA will finish when either the sequence completes or the OBA is aborted. A "seqless" (no sequence specified for the OBA) will start and end at its schedule times. Seqless OBAs can be used to keep the rover awake during a period of time (if the OBA is MUST_BE_AWAKE) or model resources that are being used in the background and are not tied to sequence execution (e.g. a PIXL overnight scan). |
| Ground (verb) |  | Run the Copilot scheduling algorithm on the COCPIT plan to find start times for the activities in the plan based on the constraints provided as well as create heating, wakeups, and shutdowns. |
| Grounded Plan |  | A COCPIT plan that has been grounded after all user constraints have been incorporated. |
|  | H |  |
| Handover (noun) |  | The transition between tactical plans onboard the spacecraft. Historically, this has occurred when the previous master sequence activates the next master sequence, or when a load-and-go master is uploaded during the uplink window while another master sequence is running. In SP operations, there will now also be handovers between M/SM and OBP control, as well as handover between OBP plan files. Each plan file specifies the handover time to transition to the next plan file, and the handover will automatically occur at that time if the rover is awake. A plan file can also be activated by ground command, which is the mechanism for transitioning from M/SM to OBP control. To transition from OBP to M/SM control, the plan will first need to be deactivated by command before the next master can take control of shutdowns. In the event that the next plan file cannot be activated (e.g. not onboard or fails validation), then a runout plan file will attempt to activate. If runout file cannot be activated either, then the current plan will be deactivated and the rover will drop into automode. When handing over between OBP plans, all uncommitted activities will be removed from the set of activities eligible for scheduling. Any activities committed (which includes those that are currently running) will persist into the next plan. This applies to runout activation as well. |
|  | I |  |
|  | J |  |
|  | K |  |
|  | L |  |
|  | M |  |
| Maintenance Heating Activity |  | The OBA representation of a warmup heater table entry that is currently in maintenance mode. Each maintenance heating activity is preceded by a preheat activity. The warmup table entry is added via the preheat activity and continues into the maintenance heating activity. The maintenance heating activity ends when there are no remaining users for the warmup table entry, and the entry is deleted when the activity ends. A maintenance heating activity may be extended to cover multiple users if there is not enough time between the activities for a full cooldown/preheat cycle, as governed by the min_cooldown_time parameter. |
| Mandatory Activity |  | Generic OBAs with a scheduling priority less than (higher priority) the max_optional_priority parameter are considered mandatory to execute. Functionally, they will have priority for scheduling compared to optional activities, and will be scheduled/executed even if doing so results in a violation of the minimum handover SOC and maximum handover delta data volume constraints on the plan. |
| Manual Activity |  | An OBA that is provided by the ground team via the plan file. For SP1 operations, this includes generic, X-band comm, UHF reliable comm, and UHF bitstream comm OBAs. See also "Generic". |
| Master/Submaster | MSM or M/SM | The conventional sequencing paradigm in use prior to Simple Planner deployment, in which activity execution is governed by a master sequence. This heritage method of operating Mars rovers has been proven across the MER, MSL, and M2020 missions. Compared to Simple Planner operations, execution timing is very deterministic, but requires large margins to account for uncertainty in environmental conditions, power modeling, and activity durations. |
| Merge Set |  | Multiple COCPIT Activity Groups chained together into a single entity. Can only be created tactically using strategically-developed Activity Groups. |
|  | N |  |
|  | O |  |
| Onboard Activity | OBA | The base planning unit for OBP FSW. The ground team provides manual OBAs via the plan file, and FSW generates additional autonomous OBAs to support the manual OBAs. The top-level activities and merge sets within COCPIT are translated into manual generic OBAs and added to the plan file for uplink to the spacecraft, and OBP FSW will place these OBAs in the schedule based on the constraints provided by the ground team and execute them. See also "Manual Activity" and "Autonomous Activity". |
| Onboard Plan File | OBPF | The command file containing the list of manual OBAs for OBP to execute, along with their attributes, resources, and constraints. This replaces the master sequence as the main command product in SP operations. |
| Onboard Planner | OBP | The FSW component of Simple Planner which conducts on-board autonomous scheduling. Fundamentally, OBP is an activity scheduler and executive. In SP operations, it replaces the master and thermal management sequences (except in the case of HGA heating). OBP moves the rover's resource management (time, power, energy, data volume, atomic resources, and sequence engines) onto the spacecraft and can react to changes in these resources as activities execute throughout the day, compared to M/SM operations where these resources must be managed via open-loop ground modeling between the decisional passes. OBP is able to autonomously generate/execute warmup heating based on environmental conditions, as well as wakeups and shutdowns based on scheduled/actual activity execution and battery SOC (i.e. add wake time for shunt mitigation). OBP is a FSW component (like Mobility and AEGIS), meaning that it can be updated without changing the monolithic FSW image. It consists of three modules: plan (the scheduler), planc (the executive), and timeline (library code for resource timeline modeling). |
| Opportunistic Activity |  | See "Optional Activity" |
| Optional Activity |  | Generic OBAs with a scheduling priority greater than (lower priority) or equal to the max_optional_priority parameter are considered mandatory to execute. They will be placed in the schedule after all mandatory activities have been placed, but not if doing so results in a violation of the minimum handover SOC or maximum handover delta data volume constraints on the plan. |
|  | P |  |
| Pin (verb) |  | Constrain the start time of an OBA to be at a specific time. |
| Place (verb) |  | See "Schedule" (verb) |
| Plan |  | A very overloaded term in Simple Planner operations. As a noun, it may refer to the COCPIT plan, the Onboard Plan File, the OBP schedule, or the Plan task in FSW. As a verb, it may refer to creating the tactical team creating the COCPIT plan. It helps to be specific when using the word "plan" in SP operations. |
| Plan Activation |  | Occurs via command or by reaching the automatic handover time to the next plan file. During plan activation, the OBP plan file being activated is validated, and its contents are added to the set of activities onboard eligible to be scheduled. When transitioning to MSM to OBP control, plan activation will deactivate all ongoing sequences (e.g. the master) once plan file validation has succeeded. |
| Plan Controller Task |  | FSW task that executes activities in the schedule produced by the Plan task. Has authority to alter the start time of activities. |
| Plan Deactivation |  | Occurs via command, when failing to activate both the next plan and the runout, when there are no remaining activities left that can be scheduled, and when a safing response runs. Plan deactivation aborts all ongoing OBAs (if not safing, this includes running their cleanups), vetoes any committed OBAs, and relinquishes OBP control of shutdowns, dropping rover into automode. |
| Plan File |  | See "Onboard Plan File" |
| Plan Setup Sequence |  | Specified in the plan file and runs as part of activation for that plan file. Plan activation waits for the setup sequence to return with success before finishing activation and scheduling the new plan file contents. Returning with failure will cause plan activation to fail. The setup sequence is intended to perform any changes to comm windows, thermal relations, or OBP-related parameters for the current plan before scheduling starts. |
| Plan Task |  | FSW task that generates schedules given operator input and current vehicle resources. This is the scheduling component of OBP. |
| Preheat Activity |  | The OBA representation of a warmup heater table entry in preheat mode, though the entry itself should transition from preheat to maintenance mode within the execution of the preheat. At the start of the OBA, the entry is added to the warmup heater table. The activity will complete at the scheduled end time if the target temperature has been reached, otherwise the activity will run long until target temperature has been achieved, delaying the start of the heated OBA(s) that require the heating. Once the preheat completes, its maintenance heating counterpart will start. |
| Priority |  | See "Scheduling Priority" |
| Push |  | Refers to an OBA's execution being delayed past its scheduled start time, such as due to dependency constraints not yet being satisfied. If an activity is pushed past a parameterized duration, the activity will be vetoed and rescheduled (this is often referred to as "push to veto"). |
|  | Q |  |
|  | R |  |
| Resource Bits |  | Constituent bits of a 32-bit value (represented by a hex code for each OBA in the plan file). Each bit corresponds to a resource claimed by an activity to help OBP encode parallelism constraints. If one activity consumes a resource, the corresponding resource bit for it is set to 1 and no other activities that also consume that resource can be scheduled in parallel. |
| Runout |  | A runout plan file is what will be activated in the event that handover to the next plan file fails (e.g. the file is not onboard or fails validation). This contains the same attributes/structure as any other plan file, including the handover time and name for the subsequent plan, giving the ops team a hook for restoring nominal plan execution on the next uplink window. OBP remains in control during OBP runout and attempts to execute whatever activities are specified in the runout plan. |
|  | S |  |
| Sampling Bridge OBA |  | It is necessary to merge SHA Shoulder / Elbow heating between the two separate OBAs that acquire the sample and process the sample and to avoid napping between these two OBAs. One way to do accomplish this is with a third, Bridge OBA, that overlaps in time between the acquire sample OBA and process sample OBA. The Bridge OBA claims SHA Shoulder / Elbow heating and requires the CPU to be on. This will result in OBP merging SHA Shoulder / Elbow heating across all three OBAs and keep the rover awake across the OBAs. |
| Schedule (noun) |  | Refers to the set of activities onboard the rover that OBP has found times to execute. |
| Schedule (verb) |  | Scheduling is performed by Copilot (the on-ground scheduler) to place activities in the COCPIT plan, as well as by OBP in flight to place activities in the schedule. The algorithms used by the two schedulers are very similar, finding places to execute activities based on the constraints provided by the tactical team as well as the modeled resources onboard the rover. Copilot scheduling is performed when the COCPIT plan is grounded. OBP scheduling triggers include plan activation, waking up after a nap, a 1-hour (parameterized) timer starting after each scheduling cycle, and whenever execution varies significantly from scheduled (activity starts early, ends early/late, vetoes, completes with failure, aborts). OBP scheduling during comm windows may be disabled (parameterized). See also "Throttling". |
| Scheduling Cycle |  | See "Scheduling Event" |
| Scheduling Event |  | Refers to the period in which OBP is scheduling activities. See "Schedule (verb)" |
| Scheduling Priority |  | An attribute of OBAs within a plan file that specifies the order in which OBP should place activities in the schedule, starting from the lowest numeric value (highest priority) and increasing. Note that within a scheduling cycle, OBP does not backtrack after it has placed an activity (i.e. after placing a high priority activity, the activity is fixed and will not be moved to make room for a lower priority activity). |
| Scheduling Window |  | A moving window starting from the current time and ending at "scheduling_window_duration" (PIE parameter) seconds in the future. OBAs become committed when entering the scheduling window. The Plan task is not allowed to schedule new activities to start during this window, as the timing of activities within this window is governed by the Plan Controller task. |
| Setup Sequence |  | See "Plan Setup Sequence" |
| Shutdown Activity |  | An autonomous activity scheduled by OBP that triggers a shutdown when it starts. |
| Simple Planner | SP | A new end-to-end autonomous on-board activity scheduling capability, comprised of the ground-based planner (Copilot) and the FSW-based On-Board Planner (OBP). These are the mission system elements which allow on-board autonomous scheduling of activities. |
| Simple Planner Phase 1 | SP1 | The first phase of Simple Planner operations, releasing the core set of ops capabilities. Also referred to as "The Accordion Planner". |
| Simple Planner Phase 2 | SP2 | The second phase of Simple Planner operations, releasing the full set of ops capabilities. Note that this does not include the full set of FSW OBP capabilities, on those that have been V&V'ed and ops has deemed valuable. Also referred to as "Full Simple Planner". |
| Snap |  | Refers to an OBA starting earlier than scheduled as soon as its dependency constraints are satisfied. Snapping can only occur if there are no activities with sequences running in parallel at the time. |
| Synthetic Activities |  | OBAs that are generated by Copilot and included in plan file, but not represented in COCPIT. For example, when a heated OBA is placed by tactical team in the COCPIT plan after handover, Copilot will instead include a heated seqless OBA in the plan file with a pinned start when heating needs to be ready by. See also "Gatekeepers" as another type of synthetic OBA. |
|  | T |  |
| Thermal Relations Binary |  | The binary-representation of the Thermal Relations Tables. In order for OBP to make use of the table, the file must be uplinked to the spacecraft's file system and then installed to NPM via command. |
| Thermal Relations Table |  | A set of tables used to generate warmup heater prescriptions for each thermal zone, based on LTST as well as zone temperature. Used by OBP to autonomously create warmup/maintenance heating activities. There are 10 tables per thermal zone: 2 may be updated tactically, 4 will be updated seasonally, and the last 4 are not expected to be updated (pan-seasonal). In order for OBP to use a thermal zone, all 10 of its tables must be installed to NPM. |
| Throttle |  | Throttling occurs when OBP scheduling is triggered several times within a short time period. To prevent FSW from being stuck in a cycle of constantly scheduling (a CPU-intensive activity), throttling prevents scheduling for a cooldown duration. The cooldown duration grows with each successive trigger up to a maximum value, until the scheduling triggers stop coming in and the throttling behavior is reset. This behavior is governed by the PLAN→THROTTLE parameter group. |
| Timeline |  | A FSW construct used by OBP to model battery SOC, RPAM peak power, change in data volume, number of sequence engines in use, resource claims in use, and awake/asleep states over the course of a schedule. |
|  | U |  |
| User |  | In the context of heating, a generic or expanding activity that requires heating is considered to be a user of the heating. Note that a single preheat+maintenance heating pair can have multiple users if they are close enough together that the maintenance heating can extend to cover both. The min_cooldown_time parameter governs the minimum time between the end of a maintenance heating activity and the start of a new preheat |
|  | V |  |
| Veto |  | If the Plan Controller is unable to start a committed OBA, the OBA will be vetoed, which gives authority of the OBA's start back to the Plan task for rescheduling. A veto will trigger a scheduling event. Generally, an OBA will be vetoed if it has not been able to start within "max_start_overdue" (PIE parameter) seconds after its scheduled start time, though there are other triggers for early vetoing, such as a dependent activity constraint not being met. Upon plan deactivation, all committed OBAs will be vetoed. |
|  | W |  |
| Wakeup Activity |  | An autonomous activity scheduled by OBP to wake the rover up. On shutdown, the rover's alarm clock will be set for the start time of the wakeup activity. |
|  | X |  |
|  | Y |  |
|  | Z |  |